Integrated Multiple Output Synchronous DC-DC Converter for Vehicle Electrical Power Supply

Abstract—This paper proposes a novel integrated synchronous buck converter for the auxiliary power supply system of vehicle, which achieves multiple independently regulated outputs with reduced switching components in comparison with the conventional separate buck converters. In order to obtain better understanding of the proposed converter, operational principle and performance characteristics of a simplified dual-output buck converter are introduced in detail, as an example. The analysis shows that zero-voltage-switching (ZVS) operation and lower conduction losses could be attained. In addition, its dynamic behavior is similar with the conventional buck converters and thus the controller design is simple. Experimental results based on a prototype circuit in which two inductors are integrated into one magnetic core to achieve further reduced cost, are demonstrated to verify the advantages. Finally, similar integrated dc-dc converters with multiple non-isolated and isolated outputs are also derived to satisfy requirements for different applications.

Index Terms— DC-DC Converter, Integrated, Multiple Output, Zero-voltage-switching (ZVS)

INTRODUCTION

W

ith the increasing concern about global warming problem and fossil fuel shortage, there is a growing interest in electric vehicles (EVs) [1-15]. In the auxiliary power supply system of EVs, multiple DC-DC converters are usually required to satisfy different load conditions, as shown in Fig. 1(a). Although employing N independent single-output dc-dc converters is an effective candidate to generate N outputs, the cost is high because the component count directly increases N times. For example, 2N switches and N inductors are needed in a separate N-output synchronous buck converter, as illustrated in Fig. 1(b). In order to reduce the overall system cost, many novel multiple-output converters [16-33] including the single-inductor multiple-output (SIMO) converters and the integrated multiple-output converters with shared switches, have been extensively studied in the recent research.

In the SIMO converters [16-29], only one inductor is needed and hence reduced number of magnetic components is achieved. Nevertheless, SIMO converters suffer from severe cross-regulation problem among different outputs in the continuous conduction mode (CCM), which results in deteriorated dynamic behavior. In order to alleviate the cross-regulation problem, improved control schemes are implemented in  [23-27].  However, the control complexity  is

(a)

(b)

Fig. 1.  Auxiliary power supply system of EVs: (a) power supply system and (b) conventional separate N-output buck converter.

increased. In [28], improved cross-regulation as well as simple control are attained with the time multiplexing control technique in discontinuous conduction mode (DCM). But the peak inductor current is large under heavy load condition, which degrades the converter performance. In order to reduce the peak inductor current, [29] adopts the freewheel switching control technique to enable the SIMO converter to operate in pseudo-continuous conduction mode (PCCM). However, an additional freewheeling switch is required, resulting in increased cost and conduction losses.

In order to eliminate the cross-regulation problem, integrated non-isolated dc-dc converters which independently regulate two output voltages with two duty-cycle, were proposed in [30-33]. In addition, reduced switches are achieved that only 3 switches are employed while 4 switches are needed in a conventional separate dual-output buck converter derived from Fig. 1(b). Therefore, lower cost as well as simple structure are obtained.

In this paper, based on the similar integrated concept, a novel integrated multiple output synchronous buck converter is proposed in Fig. 2(a), which only needs N+1 switches S0-SN. With the drive signals in Fig. 2 (b) and , switch Si (i=1, 2...N) functions as switches S2i in Fig. 1(b) and the series connection of rest switches S0, S1...Si-1, Si+1...SN operates as the switch S1i to generate N independent duty-cycles D1-DN. Therefore, in comparison with the conventional separate multiple output buck converter in Fig. 1(b), output voltages Vo1-VoN are independently regulated with reduced number of switches and associate driver circuits in the proposed converter,  and thus the overall cost is significantly reduced.

The paper is organized as follows. In order to obtain better understanding of the proposed converter, a simplified integrated dual-output buck converter are detailed introduced in the section II-IV with its operation principle, performance characteristics, design considerations and experimental results. Moreover,  in section V, isolated cells including asymmetrical flyback, forward and half-bridge are integrated with the buck converter to obtain  simultaneous multiple non-isolated and isolated outputs. Finally, conclusions are drawn in the section VI.

(a)

(b)

Fig. 2.  Proposed integrated multiple output buck converter: (a) topology and (b) drive signals.

Integrated Dual-Output Buck Converter

In this section, the simplified integrated dual-output synchronous buck converter in Fig. 3(a) are thoroughly analysed, as an example. Compared with the conventional separate dual-output synchronous buck converters with four switches, only three switches are required, contributing to reduced switches and lower cost. Besides, zero-voltage-switching (ZVS) operation of one switch and lower conduction losses could be attained. Therefore, both cost and losses are satisfactorily decreased in the proposed converter. In addition, closed-loop controller of the conventional buck converter can be directly applied to the proposed converter since their dynamic behaviours are similar, and thus the control system is also simple.

(a)

(b)

Fig. 3.  Integrated dual-output synchronous buck converter: (a) topology and (b) steady-state operating waveforms.

Operational Principle

As shown in Fig. 3(a), the proposed integrated dual-output buck converter is composed of only three switches S0-S2 and two passive filters L1-Co1 and L2-Co2. Two independent duty-cycles D1-D2 which are respectively employed to control output voltages Vo1 and Vo2, are obtained with the drive signals in Fig. 3(b). In order to simplify the analysis, several assumptions are made: (i) dead-time between switches is neglected; (ii) on-resistance Rds of all switches is assumed to be constant; (iii) inductor current iL1 is larger than iL2; (iv) switches are ideal. The steady-state operating waveforms of the proposed converter are shown in Fig. 3(b). In a switching period, the converter operation consists of three different operating intervals and the corresponding equivalent circuits are illustrated in Fig. 4.

(a)                                                (b)

(c)

Fig. 4.  Equivalent circuits at different stages: (a) stage 1 (t0-t1), (b) stage 2 (t1-t2), and stage 3 (t2-t3).

Stage 1 (t0-t1): Prior to t0, both S1 and S2 are on. At t0, S1 is turned off and S0 is turned on. Then the inductor current iL1 commutes from S1 to S0 and the drain-to-source current in S2 changes from -iL2 to iL1-iL2. In this stage, the inductor L1 is charged and the inductor L2 is discharged, as shown in .

Stage 2 (t1-t2): At t1, S2 is turned off and S1 is turned on. The drain-to-source current of S0 changes from iL1 to iL2 and the differential current -iL1+iL2 flows through S1. In this stage, the inductor L1 is discharged while the inductor L2 is charged, as shown in .

Stage 3 (t2-t3): At t2, S0 is turned off and S2 is turned on. The drain-to-source current in S1 changes from -iL1+iL2 to -iL1 and the inductor current iL2 flows through the parallel body diode of S2. In this stage, both the inductor L1 and L2 are discharged, as shown in .

From Fig. 3(b), the drain-to-source current of S1 is always less than zero and thus it operates as a synchronous rectifier. The current is2 flows through the parallel body diode of S2 at t2 when it is turned on and then naturally commutes to its mosfet channel in the next interval. Therefore, S2 is ZVS turned on.

Steady-State Characteristics

From ,  and , the inductor voltages vL1 and vL2 at different stages are obtained in TABLE I, which neglect the ripple of inductor current and thus iL1, iL2 are respectively equal to the output current Io1, Io2. Then according to the flux balance of inductors L1 and L2, voltage transfer ratios of the proposed converter are given in , which are regulated by two duty-cycle D1 and D2. With the assumption of on-resistance Rds=0.05Ω, the variations of duty-cycle D1, D2 in the prototype circuit with system parameters in TABLE II as a function of two output current Io1, Io2 are depicted in Fig. 5, which are small over whole load range. It is noteworthy that the voltage transfer ratio in  is the same as the conventional buck converter if the on-resistance Rds of switches is ignored, and D1, D2 would keep constant irrespective of the load variation.

TABLE I

Inductor Voltages at Different Stages.

|  | vL1 | vL2 |
| --- | --- | --- |
| [t0, t1] | Vin-Vo1- (2Io1-Io2)Rds | -Vo2+ (Io1-Io2)Rds |
| [t1, t2] | -Vo1- (Io1-Io2)Rds | Vin-Vo2+(Io1-2Io2)Rds |
| [t2, t3] | -Vo1- Io1Rds | -Vo2- Io2Rds |

Fig. 5.  Duty-cycle variation as a function of output current.

TABLE II

System Parameters

| Parameter | Symbol | Value | Unit |
| --- | --- | --- | --- |
| Input Voltage | Vin | 48 | V |
| Output Voltages | Vo1/Vo2 | 12/5 | V |
| Maximum Output Currents | Io1/ Io2 | 6/4.8 | A |
| Switching Period | Ts | 10 | μs |

Fig. 6.  Drive signals generation.

According to Fig. 3(b), the duty cycle Ds0-Ds2 of three switches as a function of D1, D2 are derived in . Compared with conventional separate dual-output synchronous buck converter, the proposed converter is more suitable in the application where one output voltage gain is high step-down because the narrow duty cycle of switch could be avoided. The generation of drive signals S0-S2 is simple that drive signals of S1, S2 are directly derived from the comparison between reference Vref1, Vref2 and sawtooth waveform Vt, and drive signal of S0 is the logic XOR result of S1 and S2, as illustrated in Fig. 6.

Small-Signal Model

By considering the averaged inductor voltages and the capacitor currents in each switching interval [34], the state-space model of the proposed integrated dual-output buck converter is derived in  and . The expressions A, B1, B2 and C are presented in Appendix A. From  and , the small-signal model of the proposed converter is shown in Fig. 7(a). Parameters R0, R1, R2 and e11, e12, e21, e22 are given in  .

,

(a)

(b)

(c)

Fig. 7  Small-signal mode and bode plots: (a) small-signal mode (b) Gvd11, Gvd12 and (c) Gvd21, Gvd22.

From Fig. 7(a), the output voltages  and  are determined by  and , as illustrated in . Simulation results of the bode plots of transfer function Gvd11, Gvd12, Gvd21 and Gvd22 are depicted in Fig. 7(b) and (c) with the parameters: (i) Vin = 48 V, D1=0.25, D2= 0.104, (ii) L1 = 75 μH, L2 = 47 μH, (iii) C1 = C2 = 220 μF and (iv) Ro1 = 2 Ω,  Ro2 = 1.04 Ω. From Fig. 7(b) and (c), DC gains of Gvd12 and Gvd21 are small, which can be neglected in comparison with  Gvd11 and Gvd22. Therefore, the output voltages  and  are almost independently regulated by  and , respectively. Actually, with the neglect of on-resistance Rds, R0= R1=R2=e12=e21=0, e11=Vin/D1 and e12=Vin/D2 can be obtained from  and thus the small-signal model of the proposed converter  in Fig. 7(a) is the same as that of the conventional separate two buck converter.

Comparison

Current stresses of switches in the proposed dual-output converter and the conventional two separate converters(can be derived from Fig. 1(b)) are depicted in Fig. 8 and addressed in TABLE III. Current is0 is always equal to the sum of is11 and is12 in the whole switching period. Differences only exist between is2 and is22 during the interval [t0, t1] and exist between is1 and is21 during the interval [t1, t2]. According to TABLE III, the smaller difference between iL1 and iL2 is, the lower conduction loss can be achieved in the proposed converter.

TABLE III

Comparison of Current Stresses

| [t0, t1] | is0 = iL1 | is11 = iL1, is12 = 0 |
| --- | --- | --- |
|  | is1 = 0 | is21 = 0 |
|  | is2 = iL1-iL2 | is22 = -iL2 |
| [t1, t2] | is0 = iL2 | is11 = 0, is12 = iL2 |
|  | is1 = iL2-iL1 | is21 = -iL1 |
|  | is2 = 0 | is22 = 0 |
| [t2, t3] | is0 = 0 | is11 = 0, is12 = 0 |
|  | is1 = -iL1 | is21 = -iL1 |
|  | is2 = -iL2 | is22 = -iL2 |

Comparison results among the proposed dual-output converter, conventional two separate converters and integrated dual-output converter in [30, 31] are also illustrated in TABLE IV. Switches voltage stresses and inductor currents iL1, iL2 are the same in these three converters. However, the number of switches is reduced to three and ZVS operation of one switch is achieved in the proposed converter and the integrated dual-output converter in [30, 31]. Moreover, the proposed converter can achieve lower conduction losses in the application with small difference between two output currents.

(a)

(b)

Fig. 8.  Current stresses of switches: (a) Proposed converter and (b) Conventional two separate converters.

TABLE IV

Comparison of Different Dual-Output Converters

|  | Switches |  |  |  |
| --- | --- | --- | --- | --- |
|  | Voltage Stress | Current Stress | Number | ZVS |
| Conventional | Vin | Medium | 4 | 0 |
| Integrated[30, 31] | Vin | High | 3 | 1 |
| Proposed | Vin | Low | 3 | 1 |

Design Considerations and Magnetic Integration

Switches

The voltage stresses of switches are equal to the input voltage Vin because switches are clamped to the input at turn-off. From TABLE III, current stresses of switches are obtained in , with neglect of the inductor ripple current. In the prototype circuit with system parameters in TABLE II, Mosfet IPP530N15N3 is chosen as switches.

Magnetic Integration

The requirement of two inductances L1 and L2 is given in  with the assumption that ripple inductor current  and . Generally, two magnetic cores are needed to implement two inductors. In order to achieve reduced components, two inductors L1 and L2 are integrated into one magnetic core EE-40 in the prototype circuit, as illustrated in Fig. 11(a) and (b). There are some air gaps δ1 and δ2 in the outer legs of the magnetic core, which can effectively decouple two inductors. Simulation based on finite element analysis (FEA) shows that the mutual inductance M12=0.766 μH. Therefore, the coupling coefficient  is small enough and the influence between two inductors L1 and L2 can be ignored. In addition, the magnetic induction B3 in the middle leg is small because the magnetic flux Φ3 is equal to the difference of Φ1 and Φ2 in the outer legs. Simulation results of minimum B3 at load condition Io1=6A, Io2=4.8A and maximum B3 at load condition Io1=6A, Io2=0A are respectively given in Fig. 11(c) and (d).

(a)                                                    (b)

(c)                                             (d)

Fig. 9  Magnetic integration and 3D FEA simulation: (a) scheme, (b) integrated inductors, (c) minimum B3 and (d) maximum B3.

Controller

According to section II, small-signal model of the proposed converter is similar with the conventional buck converter and hence simple proportion-integrator(PI) compensators can be employed in the closed-loop system, as illustrated in Fig. 10. The PI parameters for two control loops are given in . Experiment result of the transient response with load variation is shown in the following section that good dynamic behavior is achieved in the proposed converter.

Fig. 10  Closed-loop control system.

Experiment Verification

The experiment waveforms of drive signals vgs0,1,2, inductor current iL1,2 and drain-to-source current is0,1,2 at full load condition are shown in Fig. 11(a) and (b), which are in well coincidence with the theoretical analysis. The current is2 in the interval [t0, t1] and is1 in the interval [t1, t2] are equal to the difference between iL1 and iL2, which is small as shown in Fig. 11(b). Therefore, conduction losses can be significantly reduced at full load condition. It is noteworthy that the spike current in S0-S2 is caused by the reverse recovery of the parallel diode of S1. From Fig. 11(c), drain-to-source voltage vds2 has already decayed to zero and current is2 flows through parallel diode before S2 is turned on. Therefore, ZVS operation of S2 is achieved and switching losses are reduced. The transient response with load variation is also illustrated in Fig. 11(d). Good dynamic behavior is achieved and no cross regulation problem exists that the output voltage vo1(vo2) is almost not influenced by the variation of load current io2(io1).

(a)

(b)

(c)

(d)

Fig. 11.  Experiment waveforms:(a) drive signals vgs0,1,2 and inductor current iL1,2, (b) drive signals vgs0,1,2 and drain-to-source current is0,1,2, (c) ZVS operation of S2 and (d) dynamic behavior.

Fig. 12 shows the measured efficiency of proposed converter over the whole load range, which is higher in comparison with the conventional separate buck converters because of the soft-switching characteristic and reduced conduction losses. The efficiency improvement is remarkable at light load condition owing to the reduced switching loss which is the dominant factor of total losses.

Fig. 12  Measured efficiency.

Topology Extension

In applications with requirement of simultaneous non-isolated and isolated outputs, isolated cells including the asymmetrical flyback, forward and half-bridge can be integrated with the buck converter to achieve reduced switches. As shown in Fig. 13, N-1 isolated asymmetrical flyback outputs as well as one buck output can be easily obtained. Furthermore, asymmetrical flyback cells can be replaced by forward or half-bridge cells to achieve different isolated outputs. The drive signals of switches S0-SN are similar with that in Fig. 2 (b) to independently regulate outputs Vo1-VoN. Compared with the conventional separate dc-dc converters, reduced cost is achieved along with similar performance characteristics in the proposed integrated dc-dc converters.

Fig. 13.  Integrated dc-dc converters with multiple non-isolated and isolated outputs.

Conclusion

In this paper, an integrated synchronous buck converter using only N+1 switches to independently generate N-output voltages was proposed. Compared to the conventional N separate buck converters, the proposed converter operates similarly but achieves the advantage of reduced switches. In addition, analyses based on the simplified dual-output synchronous buck converter show that ZVS operation and possible lower conduction losses could be attained. A 12V/6A and 5V/4.8A dual-output prototype circuit was established, which integrated two inductors into one magnetic core to achieve further reduced components. From the experimental results, high efficiency as well as good dynamic behavior are obtained in the proposed converter. Finally, in order to extend to different applications, integrated converters with simultaneous non-isolated as well as isolated outputs including asymmetrical flyback, forward and half-bridge outputs, were also derived.

Appendix A

References

[1]	J. G. Pinto, V. Monteiro, H. Gonçalves, and J. L. Afonso, "Onboard Reconfigurable Battery Charger for Electric Vehicles With Traction-to-Auxiliary Mode," IEEE Trans. Veh. Technol., vol. 63, no. 3, pp. 1104-1116, Mar. 2014.

[2]	W. Cai and F. Yi, "An Integrated Multiport Power Converter With Small Capacitance Requirement for Switched Reluctance Motor Drive," IEEE Trans. Power Electron., vol. 31,no. 4,  pp. 3016-3026, Apr. 2016.

[3]	F. Akar, Y. Tavlasoglu, E. Ugur, B. Vural, and I. Aksoy, "A Bidirectional Non-Isolated Multi Input DC-DC Converter for Hybrid Energy Storage Systems in Electric Vehicles," IEEE Trans. Veh. Technol., Early Access.

[4]	T. Kim and S. Kwak, "A Flexible Voltage Bus Converter for the 48/12 Volt Dual Supply System in Electrified Vehicles," IEEE Trans. Veh. Technol., Early Access.

[5]	Y. Kim and J. Lee, "Full-Bridge+SRT Hybrid DC/DC Converter for 6.6kW EV On-Board Charger," IEEE Trans. Veh. Technol., Early Access.

[6]	C. Li, W. Huang, R. Cao, F. Bu, and C. Fan, "An Integrated Topology of Charger and Drive for Electric Buses," IEEE Trans. Veh. Technol., Early Access.

[7]	D. Chakraborty, E. Breaz, A.K. Rathore, and F. Gao, "Parasitics Assisted Soft-switching and Secondary Modulated Snubberless Clamping Current-fed Bidirectional Voltage Doubler for Fuel Cell Vehicles," IEEE Trans. Veh. Technol., Early Access.

[8]	X. Pan and A. K. Rathore, "Small-Signal Analysis of Naturally Commutated Current-Fed Dual Active Bridge Converter and Control Implementation Using Cypress PSoC," IEEE Trans. Veh. Technol., vol. 64, no. 11, pp. 4996-5005, Nov. 2015.

[9]	J. Deng, S. Li, S. Hu, C. C. Mi, and R. Ma, "Design Methodology of LLC Resonant Converters for Electric Vehicle Battery Chargers," IEEE Trans. Veh. Technol., vol. 63, no. 4, pp. 1581-1592, May. 2014.

[10]	H. Wang, S. Dusmez, and A. Khaligh, "Design and Analysis of a Full-Bridge LLC-Based PEV Charger Optimized for Wide Battery Voltage Range," IEEE Trans. Veh. Technol., vol. 63, no. 4, pp. 1603-1613, May. 2014.

[11]	R. Wai and J. Liaw, "High-Efficiency-Isolated Single-Input Multiple-Output Bidirectional Converter," IEEE Trans. Power Electron., vol. 30, no. 9, pp. 4914-4930, Sep. 2015.

[12]	C. Li, L. Herrera, J. Jia, L. Fu, A. Isurin, A. Cook, Y. Huang, and J. Wang, "Design and Implementation of a Bidirectional Isolated Cuk Converter for Low-Voltage and High-Current Automotive DC Source Applications," IEEE Trans. Veh. Technol., vol. 63, no. 6, pp. 2567-2577, Jul. 2014.

[13]	Y. Cho and J. Lai, "High-Efficiency Multiphase DC-DC Converter for Fuel-Cell-Powered Truck Auxiliary Power Unit," IEEE Trans. Veh. Technol., vol. 62, no. 6, pp. 2421-2429, Jul. 2013.

[14]	D. S. Gautam, F. Musavi, M. Edington, W. Eberle, and W. G. Dunford, "An Automotive Onboard 3.3-kW Battery Charger for PHEV Application," IEEE Trans. Veh. Technol., vol. 61, no. 8, pp. 3466-3474, Oct. 2012.

[15]	Y. Lee, A. Khaligh, and A. Emadi, "Advanced Integrated Bidirectional AC/DC and DC/DC Converter for Plug-In Hybrid Electric Vehicles," IEEE Trans. Veh. Technol., vol. 58, no. 8, pp. 3970-3980, Oct. 2009.

[16]	D. Kwon and G. A. Rincon-Mora, "Single-Inductor-Multiple-Output Switching DC-DC Converters," IEEE Trans. Circuits Syst. II, Express Briefs, vol. 56, no. 8, pp. 614-618, Aug. 2009.

[17]  A. Nami, F.Zare, A. Ghosh, and F. Blaabjerg, "Multi-output DC–DC converters based on diode-clamped converters configuration: topology and control strategy," IET Power Electron., vol. 3, no. 2, pp. 197-208, Feb. 2010.

[18]	L. Benadero, V. Moreno-Font, R. Giral, and A. E. Aroudi, "Topologies and control of a class of single inductor multiple-output converters operating in continuous conduction mode," IET Power Electron., vol. 4, no. 8, pp. 927-935, Aug. 2011.

[19]	P. Patra, A. Patra, and N. Misra, "A Single-Inductor Multiple-Output Switcher With Simultaneous Buck, Boost, and Inverted Outputs," IEEE Trans. Power Electron., vol. 27, no. 4, pp. 1936-1951, April. 2012.

[20]	A. A. Boora, F. Zare, and A. Ghosh, , "Multi-output buck-boost converter with enhanced dynamic response to load and input voltage changes," IET Power Electron., vol. 4, no. 2, pp. 194-208, Feb. 2011.

[21]	E. Babaei and O. Abbasi, "Structure for multi-input multi-output dc-dc boost converter," IET Power Electron., vol. 9, no. 1, pp. 9-19, Jan. 2016.

[22]	H. Behjati and A. Davoudi, "Single-stage multi-port DC-DC converter topology," IET Power Electron., vol. 6, no. 2, pp. 392-403, Feb. 2013.

[23]	H. Le, C. Chae, K. Lee, S. Wang, G. Cho, and G. Cho, "A Single-Inductor Switching DC-DC Converter With Five Outputs and Ordered Power-Distributive Control," IEEE Journal of Solid-State Circuits,  vol. 42, no. 12, pp. 2706-2714, Dec. 2007.

[24]	S. Carlo and S. Mukhopadhyay, "A High Power Density Dynamic Voltage Scaling Enabling a Single-Inductor Four-Output Regulator Using a Power-Weighted CCM Controller and a Floating Capacitor-Based Output Filter," IEEE Trans. Power Electron., vol. 31, no. 6, pp. 4252-4264, 2016.

[25]	Z. Shen, X. Chang, W. Wang, X. Tan, and N. Yan, "Predictive Digital Current Control of Single-Inductor Multiple-Output Converters in CCM With Low Cross Regulation," IEEE Trans. Power Electron., vol. 27, no. 4, pp. 1917-1925, Apr. 2012.

[26]	D. Trevisan, P. Mattavelli, and P.Tenti, "Digital Control of Single-Inductor Multiple-Output Step-Down DC-DC Converters in CCM," IEEE Trans. Ind. Electron., vol. 55, no. 9, pp. 3476-3483, Sep. 2008.

[27]	J. D. Dasika, B. Bahrani, M. Saeedifard, A. Karimi, and A. Rufer, "Multivariable Control of Single-Inductor Dual-Output Buck Converters," IEEE Trans. Power Electron., vol. 29, no. 4, pp. 2061-2070, Apr. 2014.

[28]	D. Ma, W. Ki, C. Tsui, and P. K. T. Mok, "Single-inductor multiple-output switching converters with time-multiplexing control in discontinuous conduction mode," IEEE Journal of Solid-State Circuits, vol. 38, no. 1, pp. 89-100, Jan. 2003.

[29]	D. Ma, W. Ki, and C. Tsui, "A pseudo-CCM/DCM SIMO switching converter with freewheel switching," IEEE Journal of Solid-State Circuits, vol. 38, no. 6, pp. 1007-1014, Jun. 2003.

[30]	E. C. dos Santos, "Dual-output dc-dc buck converters with bidirectional and unidirectional characteristics", IET Power Electron, vol. 6, no. 5, pp. 999-1009, May. 2013.

[31]	P. Kumar and M. Rojas-Gonzalez, "Novel 3-switch dual output buck voltage regulator," in Proc. IEEE Appl. Power Electron. Conf. Expo., 2006, pp. 467-473.

[32] G. Chen, J. Dong, Y. Deng, X. He, and Y. Wang, "Integrated dual-output synchronous DC-DC buck converter," in Proc. Energy Convers. Cong. Exposition, 2015, pp. 1528-1533.

[33]	O. Ray, A. P. Josyula, S. Mishra, and A. Joshi, "Integrated Dual-Output Converter," IEEE Trans. Ind. Electron., vol. 62, no. 1, pp. 371-382, Jan. 2015.

[34]  R. W. Erickson and D. Maksimovic, Fundamentals of Power Electronics, 2nd ed. Norwell, MA, USA: Kluwer, Jan. 2001.