Are cleaner cooking solutions clean enough? A systematic review and meta-analysis of particulate and carbon monoxide concentrations and exposures

Daniel Pope1, Michael Johnson2, Nigel Fleeman1, Kirstie Jagoe2, Rui Duarte1, Michelle Maden1, Ramona Ludolph3, Nigel Bruce1, Matthew Shupler1, Heather Adair-Rohani3, Jessica Lewis3

- University of Liverpool

- Berkeley Air Monitoring Group, Berkeley, CA, USA

- Department of Environment, Climate Change and Health, World Health Organization, Geneva, Switzerland

Abstract

Background: Globally, approximately 3 billion primarily cook using inefficient and poorly vented combustion devices, leading to unsafe levels of household air pollution (HAP) in and around the home. Such exposures contribute to nearly 4 million deaths annually (WHO 2018a, 2018b). Characterizing the effectiveness of interventions for reducing HAP concentration and exposure is critical for informing policy and programmatic decision-making on which cooking solutions yield the greatest health benefits.

Methods: This review synthesizes evidence of in-field measurements from four cleaner cooking technologies and three clean fuels, using field studies aimed at reducing HAP concentration and personal exposure to health damaging pollutants (particulate matter (PM2.5) and carbon monoxide (CO)). Fifty studies from Africa, Asia, South and Latin America, provided 168 estimates synthesized through meta-analysis.

Review Results: For PM2.5 kitchen concentrations, burning biomass more cleanly through improved combustion stoves (ICS) with (n=29; 63% reduction) or without (n=12; 52%) venting  (through flue or chimney) and through forced-draft combustion (n=9; 50%) was less effective than cooking with clean fuels including ethanol (n=4; 83%), liquefied petroleum gas (LPG) (n=11; 83%) and electricity (n=6; 86%). Only studies of clean fuels consistently achieved post-intervention kitchen PM2.5 levels at or below the health-based WHO interim target level 1 (WHO-IT1) of 35g/m3. None of the advanced combustion stoves (gasifiers) achieved WHO-IT1, although no evidence was available for pellet fueled stoves. For personal exposure to PM2.5, none of the ICS (n=11) were close to WHO-IT1 whereas 75% (n=6 of 8) of LPG interventions were at or below WHO-IT1. Similar patterns were observed for CO, although most post-intervention levels achieved the WHO 24hr-guideline level.

Discussion: While clean cooking fuel interventions (LPG, electric) significantly reduce kitchen concentrations and personal exposure to PM2.5 in household settings, stove stacking and background levels of ambient air pollution, have likely prevented most clean fuel interventions from approaching WHO-IT1. In order to maximize health gains, a wholistic approach jointly targeting ambient and household air pollution should be followed in lower-and-middle income countries.

Background

Globally, some 3 billion people burn solid fuels (e.g., wood, dung, charcoal) in inefficient and poorly vented combustion devices such as three-stone-fires or rudimentary stoves as their primary means of cooking (Bonjour et al 2013, IEA et al 2020). The resulting household air pollution (HAP) contributes to over 3.8 million premature deaths per year and almost 127 million disability-adjusted life-years from heart disease, stroke, chronic obstructive pulmonary disease and lung cancer, making it a leading risk factor for global morbidity and mortality (WHO 2018a, 2018b), as well as contributing to global climate warming (Lacey et al 2017).

In recognition of the need to support countries tackle the high disease burden attributed to polluting household energy use, the World Health Organization (WHO) issued the WHO Guidelines for indoor air quality: household fuel combustion (WHO 2014) (hereafter the WHO Guidelines). These WHO Guidelines provide normative recommendations in the form of particulate matter (PM2.5) and carbon monoxide (CO) emission-rate performance targets for household energy devices (e.g., cookstoves) to achieve the WHO air quality annual and interim-1 PM2.5 guideline levels of 10 g/m3 (0.01 mg/m3) and 35 g/m3 (0.035 mg/m3)  respectively, and the 24-hour average air quality CO guideline of 7 mg/m3 (WHO 2006, 2010).

HAP concentrations of fine particulate matter are often several times higher than WHO Guidelines, with global estimates for 24 hour mean kitchen concentrations in wood-using homes ranging from ~500-1000 g/m3 (Balakrishnan et al 2014, Pope et al 2017). The interim PM2.5 air quality target was developed to provide an incremental step for air quality improvements in settings where it may be unrealistic to achieve guideline levels quickly. Considering the very high-levels of PM2.5 associated with inefficient cooking, the WHO interim target-1 (WHO IT-1) is referenced throughout this analysis.

# Policies and programs supporting the promotion, sales and dissemination of lower emissions stoves and fuels have sought to mitigate this health burden. The policies and interventions range from large national efforts such as India’s Pradhan Mantri Ujjwala Yojana program to increase access and use of liquified petroleum gas (LPG) (Dabadge 2019, Tripathi and Sagar 2019), to artisan stove-makers building and selling relatively simple stoves to local populations (Hyman 1987, Orr et al 2015). Characterizing the effectiveness of these interventions on reducing HAP and associated exposures is critical for understanding which solutions are likely to result in the greatest health benefits. With increasing international focus on the importance of meeting Sustainable Development Goals 3 (ensure healthy lives and promote wellbeing for all at all ages) and 7 (universal access to affordable, reliable, modern energy by 2030) to address the substantial global health burden from reliance on polluting cooking, this review provides timely and crucial evidence.

Importantly, the path to clean household energy will include transitional steps as the cleanest options may not be feasible in many contexts due to infrastructure, resource, or other access constraints. In these cases, the cleanest possible solutions should be prioritized (WHO 2014). The impact of multiple stove use, or stove “stacking”, can have dramatic impacts as even nominal traditional stove use can substantially elevate HAP concentrations, and thus displacement of high emission stoves is as critical as the performance of the “cleaner” technology or fuel (Johnson and Chiang 2015). Synthesizing the HAP and exposure evidence associated with household energy transitions is needed to inform smart policy, investment, and programmatic decisions and, in turn, expedite attainment of public health benefits from clean cooking.

While there have been efforts to systematically review and synthesize the effectiveness of household energy interventions (Balakrishnan et al 2014, Pope et al 2017, Quansah et al 2017), more evidence of different and promising interventions has recently become available. Newer technologies which aid combustion with forced-draft and/or pellet fuels, for example, have been introduced as interventions in sub-Saharan Africa, Asia, and elsewhere (e.g. (Champion and Grieshop 2019, Hill et al 2019, Nightingale et al 2018). Additionally, interventions with gas and liquid fuels such as ethanol and LPG have also been evaluated (e.g. (Alexander et al 2018, Bruce et al 2018, Clasen 2020, Delapena et al 2018)), in addition to emerging studies of cooking with electricity providing more evidence of what exposure reductions can be achieved with these clean burning options (references).

This paper explicitly builds on the systematic review of household energy interventions presented in Pope et al. (2017), augmenting the dataset to include these new studies. The Pope et al. 2017 review concentrated on available evidence for solid fuels stoves with very limited information on clean fuels technologies, and performed a meta-analysis of the PM2.5 and carbon monoxide (CO) kitchen concentrations and personal exposures associated with those interventions. Here we add new research to summarize the evidence of the impact of various categories of stove and fuel combinations on kitchen concentrations and personal exposures of PM2.5 and CO. This review further establishes evidence for the interventions which have a longer history of evaluation (e.g. chimney and natural draft wood stoves (Kirby et al 2019, Young et al 2019)), and updates the synthesis to provide valuable new information on current and potentially more promising intervention scenarios such as LPG, alcohol and electric stoves.

Methods

This systematic review and meta-analysis adheres to the PRISMA guidelines for reporting and provides a comprehensive assimilation and synthesis of the current evidence base for field-based studies of household air pollution (PM2.5 and CO) from cleaner cooking technologies and fuels in lower-and-middle-income-country (LMIC) settings. The focus of the review is on the “real-life” effectiveness of interventions, as distinct from studies conducted in artificial contexts such as laboratory-based studies based on emissions, field-based controlled cooking tests, or trials instructing exclusive use of an intervention. The review focussed on published and unpublished literature from 1998 to 2019, representing 30 years of intense activity in the field of household air pollution and prevention strategies.

A comprehensive search strategy was employed with searches across (i) the main bibliographic databases (PubMed, Ovid (Medline) and Scopus), (ii) the grey literature (including environmental databases (Environment Complete, GreenFILE) and Google Scholar) and specialist relevant websites, (iii) personal communication with stakeholders and research leads and (iv) reference tracking of all included studies. All searches were conducted on 16 July 2020.

Search terms were refined for each bibliographic database and included a combination of index terms and free-text words. For Google Scholar, search terms included a combination of each stove/ fuel type with “indoor air” or “household air” and the first 100 ‘hits’ were reviewed. See supplementary information for the search terms used.

## Study Eligibility

Population:

Populations from LMICs who predominantly rely on biomass, coal or kerosene as the primary baseline household energy source. All geographical contexts (urban, peri-urban or rural) were eligible. For personal exposure to household air pollution, measurements from both adults and children were included.

Interventions:

The review broadly grouped interventions into cleaner cooking technologies and clean fuels, classified into 7 categories. Cleaner cooking technologies included (i) ‘improved’ solid fuel stoves without venting (e.g. chimneys), (ii) ‘improved’ solid fuel stoves with venting, (iii) ‘complex’ interventions including multiple components (e.g. stove modifications, smoke hoods etc), and (iv) advanced combustion solid fuel stoves (e.g. stoves with a fan to aid combustion or that use a modified biomass fuel (e.g. pellets)). Clean fuels included (i) alcohol fuels, (ii) gas (LPG, biogas, natural gas) and (iii) electricity.

Comparison:

Solid fuels, biomass or kerosene used for cooking in open fires or traditional stoves.

Outcome:

Quantitative measurements of HAP containing respirable particulate matter (PM < 10 microns) and/or CO. Measurements needed to represent at least one full day of cooking (sampling HAP for at least 24 hours (+/- 2 hours)); studies with less than 22 hours duration were excluded as not representing daily time-weighted averages. For both pollutants, measurement of concentrations (levels in kitchens) and personal exposure (measured in adults and/ or children) were included. For each study/ intervention it was therefore possible to have up to four estimates of HAP: kitchen PM, kitchen CO, personal PM and personal CO.

Eligible study designs included randomized controlled trials (RCTs), before-and-after studies with or without a control group and cross-sectional study designs. For before-and-after studies, only intervention group data were used due to the majority not having a control comparison group.

Study selection

Titles and abstracts and literature identified from unpublished sources (grey literature and websites) were checked for relevance by two of the review team (NF, RL). The full texts of those potentially meeting the inclusion criteria were screened by four of the review team (NF, RL, KJ, MJ) and independently assessed by an additional reviewer (DP). Those clearly not meeting the inclusion criteria were excluded. At each stage of filtering 20% verification was undertaken independently by two reviewers with any discordant decisions being reconciled amongst the review team.

Data extraction was conducted using bespoke pre-piloted forms recording author, year, country, season (where applicable), baseline stove and fuel, intervention stove and fuel, study design, duration of follow up, number of homes assessed, analytical or sampling approach, sampling period, baseline and post-intervention data on kitchen concentrations of and personal exposure to PM and CO (mean and standard deviation, where available). One reviewer (one of NF, RL, KJ and MJ) completed each data extraction form with 20% verification through double extraction with another reviewer- discrepancies being resolved through discussion.

Quality appraisal was assessed using tools developed by the Department of Public Health and Policy, University of Liverpool (Liverpool Quality Assessment Tools (LQATs)) (Pope et al 2016). The tools have been used extensively across a wide range of health outcomes (Bruce et al 2015, Garn et al 2017, Ronzi et al 2018, Rehfuess et al 2013) and were the quality assessment instrument selected for systematic review chapters informing the WHO Guidelines (Diekman et al 2014, Bruce et al 2014, WHO 2014). The LQATs allow appraisal of multiple study designs and assign a star rating for methodological quality relating to reduced risk of bias (selection, response, follow-up and measurement), risk of confounding (balance of randomisation and adjustment) and increased accuracy of measurements of exposure and outcome. The star rating can then be used to assess a quality percentage based on the number of awarded stars out of the total available for a specific design. Quality appraisal was undertaken independently by two reviewers (NF, RL) for each included study.

## Statistical Analysis

Meta-analyses were conducted for the four distinct HAP outcomes: kitchen PM concentrations (in mg/m3), kitchen CO concentrations (in ppm), personal PM exposure (mg/m3) and personal CO exposure (ppm). Pooling of data required both estimates (means and standard deviations) of baseline concentrations/ exposure and intervention concentrations/ exposure. Standard deviations were calculated from confidence intervals or p-values if they were not readily available. Medians and geometric means were infrequently reported by the studies and were not included in the synthesis, despite often skewed distributions. Generic inverse variance meta-analyses using the Cochrane Collaboration’s RevMan software version 5 (http://www.cc-ims.net/RevMan) were used to obtain pooled absolute mean differences (e.g. intervention levels minus baseline levels) and confidence intervals for each outcome. Statistical heterogeneity for each meta-analysis was assessed through the I-squared test with random-effects meta-analyses (der Simonian and Laird 1986) being used for a test value greater than 20%.

In total seven meta-analyses were possible for each of the four HAP outcomes based on cleaner technology (n=4) and clean fuel (n=3) intervention categories, where data was sufficient for pooling. Each meta-analysis was presented in the form of a Forest Plot and was assessed for potential publication bias through measurement of funnel plot asymmetry (this was done graphically and statistically with Begg’s (Begg 1994) and Egger’s (Egger 1997) hypothesis testing).

Additional quantitative syntheses of intervention impacts were conducted including (i) calculations of percentage reductions in HAP (based on weighted mean reductions of the four HAP outcomes) for each of the 7 intervention groups and (ii) plots of baseline (no intervention) and intervention levels of the four HAP outcomes against a reference for the WHO Air Quality Guideline level for PM2.5 (Interim-Target 1 35 g/m3) and for CO (24hr target of 7 mg/m3 converted to a value of 6.11 ppm using the formula ppm = (mg/m3 value) x (24.45) / (molecular weight = 28.01), and assuming a temperature of 25°C and a pressure of 1 atmosphere) for each of the 7 intervention groups.

# Review Results

The systematic review process yielded a total of 50 studies (168 estimates) to be included in the evidence synthesis (Figure 1). The number of studies included in this review (19 additional studies (59 estimates)) increased by 60% from the previous review by Pope et al (2017) (31 studies (109 estimates)).

In terms of the four HAP outcomes, for kitchen concentrations there were 39 studies (73 estimates) with PM2.5 and 32 studies (59 estimates) with CO; and for personal exposure there were 13 studies (20 estimates) for PM2.5 and 12 studies (16 estimates) with CO. The study characteristics for each intervention type with information relating to the context of HAP measurement including country, study season, design, duration, parameters measured and quality assessment are presented in Tables 1-7.

Results from the meta-analyses are displayed according to the four primary HAP outcomes, stratified by intervention category, in Figures 2-5. The impacts on the four HAP outcomes of the cleaner cooking technologies and fuels are described below for each intervention category.

Cleaner cooking technologies

## 1. ‘Improved cookstoves’ without venting (ICS – no vent)

Nine studies (six in Africa and three in Asia) measured impacts of improved cookstoves using standard combustion (i.e. not enhanced by forced draught or gasification), and without venting. These mainly used rocket-type combustion chambers, although some used these designs with a grate, Table S1.

### Kitchen concentrations: particulate matter

Seven studies (12 estimates) measured kitchen PM, 6 before-and-after designs and one RCT. The weighted mean levels were 0.750 mg/m3 at baseline and 0.361 mg/m3 post-intervention, with a weighted mean reduction of 51.9%, Figure 6. Moderate heterogeneity (I2 = 47%) was observed and random effects meta-analysis yielded a pooled effect estimate of -0.23 mg/m3 (95% CI -0.34, -0.12), Figure S1(a). Possible publication bias was indicated by both the Begg test (p=0.099) and the Egger’s test (0.005), Figure S1(b).

### Kitchen concentrations: carbon monoxide

Eight studies (13 estimates) measured kitchen CO, seven before-and-after designs and one cross-sectional design. Weighted means were 11.1 ppm at baseline and 7.0 ppm post-intervention; weighted mean reduction of 36.1%, Figure 6. The pooled estimate in fixed effect meta-analysis (I2 = 0%), was -3.3 ppm (95%CI -4.5, -2.0), Figure S2(a). There was no evidence of publication bias from Begg’s or Egger’s tests (p>0.1), Figure S2(b).

### Personal exposures: particulate matter and carbon monoxide

No studies provided estimates of personal exposure to particulate matter whereas two studies provided two estimates of exposure to CO (statistically non-significant pooled reduction of -1.7 ppm (95% CI -4.7, 1.3)).

## 2. “Improved cookstoves” with venting (ICS – vent)

Overall 28 studies - 1, 12, and 15 conducted in Africa, Asia and Latin America respectively - concerned improved cookstoves with venting. Stove models comprised various fixed stove types made of mudbrick (e.g. ENPHO stove), metal (e.g. rocket stove) or brick (e.g. plancha stove), Table S2.

### Kitchen concentrations: Particulate matter

Fifteen before-and-after studies (19 estimates) and eight cross-sectional studies (10 estimates) measured kitchen PM. The weighted (for sample size) mean levels for kitchen PM were 0.89 mg/m3 at baseline and 0.33 mg/m3 post-intervention, a weighted mean percentage reduction of 63.3% (Figure 6.1).

With an I2 value of 86% (p<0.005), random effects meta-analysis was conducted with a pooled effect estimate of -0.45 mg/m3 (95% CI: -0.56, -0.34), Figure S3a. Figure S3b shows the funnel plot. Tests of funnel plot asymmetry were of borderline significance for Begg’s test (p=0.029), and significant for Egger’s test (p<0.0005), suggesting publication bias (likely in the direction of smaller studies reporting a larger reduction in PM). Analysis stratified by study design showed this bias to be significant for before-and-after studies (Begg’s p=0.019; Egger’s p=0.001) but not for cross-sectional studies (Begg’s p=0.210; Egger’s p=0.097).

### Kitchen concentrations: carbon monoxide

Twelve before-and-after studies (17 estimates), eight cross-sectional studies (9 estimates) and 1 randomised controlled trial (1 estimate) measured kitchen CO. The weighted mean levels were 11.9 ppm at baseline and 4.2 ppm post-intervention, with a weighted mean percentage reduction of 64.4%, Figure 6.2.

The pooled effect estimate in random effects meta-analysis was -5.62 ppm (95% CI: -7.09, -4.16), Figure S4(a). Tests for funnel plot asymmetry were non-significant for Begg’s test (p = 0.182), but significant for Egger’s test (p=0.012) (Funnel plot Figure S4(b)), potential publication bias being in the direction of smaller studies reporting larger reductions in CO. Publication bias was significant for before-and-after designs (Begg’s p=0.127; Egger’s p=0.025) but not for cross-sectional studies (Begg’s p=0.466; Egger’s p=0.114).

Personal exposures: particulate matter

Seven studies, 4 before and after studies and 3 cross-sectional studies, provided nine estimates of personal PM. The weighted mean levels were 0.697 mg/m3 at baseline and 0.171 mg/m3 post-intervention, with a weighted mean reduction of 75.5%, Figure 6.3.

There was significant statistical heterogeneity (I2 = 100% (p<0.0005)), and random effects meta-analysis yielded a statistically non-significant pooled estimate for absolute reduction in personal PM of 0.28 mg/m3 (95% CI: -0.67, 0.11), Figure S5(a). There was no statistical evidence of publication bias according to Begg’s (p=0.175) and Egger’s (p=0.404) tests, Figure S5(b).

Personal exposures: carbon monoxide

Eight studies measured personal CO: one RCT (two estimates), three before-and-after studies (four estimates), and four cross-sectional studies (5 estimates). The weighted mean levels were 3.6 ppm baseline and 1.6 ppm post-intervention; weighted mean reduction of 55.1%, Figure 6.4.

Statistical heterogeneity was observed (I2 = 84% (p<0.0005)) and the pooled effect estimate in random-effects meta-analysis was -1.36 ppm (95% CI: -1.90, -0.81), Supplementary Figure S6(a). There was no statistical evidence of publication bias in studies of personal CO according to Begg’s (p=0.169) and Egger’s (p=0.264) tests, Figure S6(b).

## 3. Complex (multi-facetted) interventions

Two before-and-after studies in Kenya and Sudan provided estimates on the impact of multi-component interventions (combinations of a hood with flue venting outside the house, 'improved' stove, fireless cooker and LPG, determined according to user preferences) (Table S3) including 2 estimates for kitchen concentrations of PM2.5 (pooled reduction of -0.23 mg/m3 (95% CI -0.36, -0.10) with a 44.8% reduction in weighted mean concentration) and 3 estimates for kitchen concentrations of CO (pooled reduction of -2.2 ppm (95% CI -7.6, 3.2) with a 45.4% reduction in weighted mean concentration), Tables 1 and 2.

## 4. Advanced combustion (“gasifier”) solid fuel stoves

Four studies (3 conducted in India and 1 in Laos) addressed impacts of advanced combustion solid fuel stoves, all before-and-after studies. Stove models comprised free convection-, forced convection- and forced-convection combined with optimized fuel stoves, Table S4.

### Kitchen concentrations: particulate matter

The four studies provided nine estimates of kitchen PM2.5 with weighted means of 0.611 mg/m3 at baseline and 0.308 mg/m3 post-intervention, resulting in a weighted average PM reduction of 49.5% (Figure 6). Fixed effect meta-analysis (I2 = 0%), yielded a pooled effect estimate of -0.23 mg/m3 (95% CI: -0.32, -0.15), (Figure S7a). No evidence of publication bias was observed with Begg’s (p=1.0) and Egger’s (p=0.458) tests, Figure S7b.

Kitchen concentrations: carbon monoxide

For kitchen CO, three studies provided 7 estimates of effect. The weighted mean values were 10.7 ppm at baseline and 5.4 ppm post-intervention, with a weighted average reduction of 49.5%, Figure 6. In random effects meta-analysis (I2 = 35%), the statistically non-significant pooled estimate was -2.0 ppm (95%CI: -4.0, 0.1) Figure S8a. There was no evidence of publication bias with Begg (p=0.368) and Egger’s (p=0.127) tests, Figure S8b.

Clean fuels

## 5. Alcohol based fuels (i.e. Ethanol)

Two before-and-after studies conducted in Nigeria, Ethiopia and two regions of Madagascar addressed impacts of ethanol on kitchen concentrations of PM2.5 (4 estimates) and CO (4 estimates); none measured personal exposure to HAP, Table S5.

### Kitchen concentrations: particulate matter

For kitchen PM, weighted means were 0.72 mg/m3 at baseline and 0.12 mg/m3 post-intervention, with a weighted mean reduction of 82.8% (Figure 6).

In random effects meta-analysis the pooled effect estimate was -0.58 mg/m3 (95%CI: -0.92, -0.24), with considerable statistical heterogeneity (I2=82%), Figure S9a. Tests for publication bias were non-significant for Begg’s test (p=0.308), but marginally significant for Egger’s test (p=0.056).

### Kitchen concentrations: carbon monoxide

For kitchen CO, weighted means were 33.0 ppm at baseline and 5.9 ppm post-intervention; the weighted average reduction was 82.2%, Figure 6. The high baseline CO values (range 15.0 to 70.5 ppm) are likely to reflect the widespread use of charcoal in these study communities.

In random effects meta-analysis, the pooled estimate was -27.3 ppm (95%CI: -41.5, -13.1), with significant heterogeneity (I2=83%), Figure S10a. Tests for publication bias were non-significant for Begg’s (p=0.089) and Egger’s (p=0.161) tests, Figure S10b.

## 6. Liquified Petroleum Gas (LPG)

Ten studies (5 cross-sectional, 2 before-and-after and 3 randomised controlled trials) assessed the impacts of cooking with LPG on concentrations of and exposure to HAP, Table S6. Study locations included Cameroon, Nigeria, Paraguay, Nepal, Ghana and India.

### Kitchen concentrations: particulate matter

For kitchen PM2.5 (11 estimates), weighted means were 0.65 mg/m3 at baseline and 0.11 mg/m3 post-intervention, with a weighted mean reduction of 83.3% (Figure 6).

In random effects meta-analysis the pooled effect estimate was -0.52 mg/m3 (95%CI: -0.650, -0.390), with significant statistical heterogeneity (I2=79%), Figure S11a. Tests for publication bias were non-significant for both Begg’s (p=0.169) and Egger’s (p=0.217) tests, Figure S11b.

### Kitchen concentrations: carbon monoxide

For kitchen CO (3 estimates), the weighted means were 14.0 ppm at baseline and 2.7 ppm post-intervention, with a weighted mean percentage reduction of 85.7%, Figure 6. Random effects meta-analysis (I2=83%) yielded a pooled effect estimate of -11.6 ppm (95%CI: -17.5, -5.8), Figure S12.

### Personal exposure: particulate matter and carbon monoxide

Whilst 6 studies provided 8 estimates of the impact of cooking with gas on personal exposure to PM2.5, only one measured CO (mean=0.68ppm; sd=0.32). Weighted mean levels of PM2.5 were 0.58 mg/m3 at baseline and 0.28 mg/m3 post-intervention, with a weighted mean reduction of 74.3% (Figure 6).

In random effects meta-analysis the pooled effect estimate was -0.10 mg/m3 (95%CI: -0.14, -0.06), with significant statistical heterogeneity (I2=84%), Figure S13a. Tests for publication bias were non-significant for the Begg test (p=0.536) and Egger’s test (p=0.066), Figure S13b.

## 7. Cooking with electricity

Five studies (all cross-sectional) examined an intervention using electricity for cooking providing 6 estimates for kitchen PM, 2 estimates for kitchen CO and one for personal PM (with no estimates for personal CO), Table S7. Study locations included Nepal, Paraguay and Nigeria.

### Kitchen concentrations: particulate matter

For kitchen PM, weighted means were 0.60 mg/m3 at baseline and 0.08 mg/m3 post-intervention, with a weighted mean reduction of 86.2% (Figure 6). In random effects meta-analysis (I2=95%) the pooled effect estimate was -0.45 mg/m3 (95%CI: -0.77, -0.14), Figure S14a. Tests for publication bias were non-significant for Begg’s (p=0.452) and Egger’s (p=0.292) tests, Figure S14b.

### Kitchen concentrations: carbon monoxide

For kitchen CO, the weighted means were 10.3 ppm at baseline and 0.6 ppm post-intervention, with a weighted mean percentage reduction of 94.6%, Figure 6. Random effects meta-analysis (I2=97%) yielded a pooled effect estimate of -9.5 ppm (95%CI: -27.6, -8.5), Figure S15.

Discussion

This systematic review synthesises data obtained from studies of estimates for concentrations of and personal exposure to respirable PM and CO. Eligible studies included both cleaner biomass burning stoves (with and without venting, and stoves that include more advanced combustion) and those with clean fuels (ethanol, LPG and electricity).

# Intervention impacts on kitchen and personal PM and CO according to WHO guidelines

Figures 6 to 9 show the effects of the cleaner cookstove and clean fuel interventions for the seven intervention categories for the four HAP outcomes and compared against WHO Air quality guidelines: Global Update 2005 (AQG) reference levels for PM2.5 and the 2009 WHO Guidelines for indoor air quality: selected pollutants for CO (WHO 2006, 2010).

For kitchen concentrations of PM2.5, all intervention types, and most individual studies, resulted in statistically significant reductions (Figure 2) and observed large percentage and absolute reductions, with only a small number reporting minor reductions or even increases. However, it is notable that only interventions involving very clean fuels (particularly for LPG stoves (11 estimates) and electric stoves (6 estimates)) consistently achieved mean post-intervention levels near or below the WHO AQG IT-1 level (Figure 6). Most of the interventions involving biomass stoves were at much higher PM2.5 kitchen concentrations than WHO IT-1. Among solid fuel stoves, those interventions with chimneys had lower PM2.5 concentrations than those without. None of the advanced combustion stoves (gasifiers) achieved WHO IT-1, although these only included stoves with forced draft combustion (the authors were not able to find any field based data from pellet fuel gasifier stoves). Kitchen PM levels from ethanol cookstove interventions were in general much lower than biomass cookstoves, but above the WHO AQG IT-1 PM2.5 target.

Similar results were observed for studies measuring concentrations (kitchens) of CO, with some notable exceptions. Like PM2.5, pre-intervention levels of CO showed a great deal of variability with many exceeding the WHO 24-hour guideline value (Figure 7). Post-intervention kitchen concentrations of CO were significantly reduced for both chimney and non-chimney cleaner biomass stoves, as well as ethanol and LPG clean fuel stove interventions (Figure 3). Chimney stoves had the best performance in reducing CO among the solid fuel stoves, and ethanol from the clean fuels. In contrast to the findings for kitchen concentrations of PM (11 out of the 73 cleaner technology and clean fuel interventions achieving the WHO-IT1 Guideline level (Figure 6)), the majority of all types of interventions achieved CO levels at or below the WHO 24-hour Guideline (51 of 59 interventions).

For personal exposure to household air pollution, baseline levels tended to be lower than for kitchen concentrations for both PM2.5 (Figure 8) and CO (Figure 9). For personal exposure to PM2.5, whilst the data was more sparse, only the interventions involving cooking with advanced combustion cookstoves, LPG or electricity resulted in a statistically significant pollutant reduction (Figure 4), and only LPG interventions consistently achieved levels below or near the WHO IT-1 AQG (Figure 8). All but one of the post intervention levels for LPG were close to or below the WHO IT-1 level of 35µg/m3 for PM2.5, with a 74% (61-83%) reduction observed to a post-intervention level of 58 µg/m3. Excluding the outlier study with an intervention estimate of 195 µg/m3 led to a post intervention average below WHO IT-1 of 33 µg/m3 and a pooled reduction of 78.1%).) For chimney stoves a 76% (19-87%) reduction was observed to a post-intervention level of 171 µg/m3 and for advanced combustion stoves a 20.3% (increase of 12.5% to reduction of 39.5%) reduction was observed to a post-intervention level of 105 µg/m3 (Figure 6).

For personal exposure to CO, chimney stoves, advanced combustion biomass stoves, and LPG interventions resulted in statistically significant reductions. All interventions (both cleaner biomass burning technologies and clean fuels) resulted in levels of personal exposure below the WHO 24-hour guideline level.

Implications for maximising health gains

Combustion derived exposure to respirable PM from active and second-hand smoking, HAP and ambient air pollution have been associated with disease outcomes (pneumonia in children and chronic obstructive pulmonary disease, lung cancer, stroke and ischaemic heart disease) through published exposure-response functions (Burnett et al 2014, 2018, HEI 2020). These mostly non-linear functions (not including lung cancer that appears to have a linear exposure-response relationship) highlight that the risk of disease declines more steeply at lower levels of exposure relative to higher levels.  For this reason it is the post-intervention levels of PM that are particularly important when considering risk on health rather than absolute or relative reductions of HAP. Whilst post-intervention levels varied substantially between interventions, most of those involving cleaner burning biomass stoves were close to the WHO annual PM2.5 IT-1 of 35 µg/m3. Epidemiological studies and related dose-response curves suggest that to have a meaningful positive impact on the burden of disease from household air pollution, interventions need to be consistently at or below the interim guideline level (Burnett et al 2014, WHO 2006). Only clean fuels interventions (studies of ethanol, LPG and electricity) reported mean post-intervention concentrations of PM2.5 close to or at the IT-1 levels (all below 100 µg/m3). For the limited number of biomass burning chimney stoves that achieved post intervention levels to the IT-1 target, further examination of these technologies and the circumstances of their use is warranted (e.g. (Young et al 2019, Terrado et al 2005)), including their impacts on fuel savings and the environment (Johnson et al 2009, Berrueta et al 2008, Granderson et al 2009).

There are a variety of factors that can contribute to reductions in emissions that affect post-intervention levels with a key influence being the use of multiple fuels and stoves in parallel to the intervention to meet different cooking and home energy needs (Quinn et al 2018, Ochieng et al 2020, Ruiz-Mercado and Masera 2015). This fuel- and/or stove ‘stacking’ is common practice and is increasingly being reported in more recent measurement studies. Considering the substantial impact that ‘stacking’ can have on the level of emissions and exposure in the home, ongoing (and future) efforts to design and promote interventions to reduce HAP and will need to account for these practices to maximize health benefits, for example by understanding and addressing the reasons why people stack and promoting combinations of cleaner interventions. In addition, the effect of lighting, space heating and other combustion sources in and around the home, as well as pollution from other homes and sources in the community, while rarely quantified in evaluation studies, contribute to the observed pre- and post-intervention concentrations and exposures. These factors highlight the need for a comprehensive solution to the HAP problem, with interventions to clean up cooking as well as other household energy uses; this is likely to be more effective if targeted at the community level rather than single households.

Strengths and limitations

This review represents a comprehensive appraisal of the current evidence base for quantified impacts on key health damaging pollutants of all cleaner and clean cooking solutions tested in field based conditions.  With a 60% increase in the number of HAP measurement studies within a three-year period, this meta-analysis draws on a wide range of published and unpublished sources including appraisal of specialist websites, conference proceedings and through direct communication with lead researchers in the clean cooking sector. Given the wealth of quantitative data available representing 7 distinct intervention categories with impacts on both concentrations (kitchen) and exposure (cooks and children, where available) to PM2.5 and CO,  a variety of approaches have been used to present results from the review using both meta-analytical representation (e.g. forest plots) as well as referencing impacts for users’ needs (i.e. relating values for baseline and post-intervention impacts to WHO AQG values). As one would expect, there is substantial heterogeneity between the included studies in relation to levels of PM2.5 and CO, intervention type, context, circumstances of intervention use and employed methodology. For this reason the synthesis is presented for both individual studies and pooled across studies, allowing consideration of both general conclusions as well as context- and intervention-specific messages.

Another strength of this review is that it considers kitchen concentrations and exposure measurements from real-life settings in which other stoves, heaters, and lamps may be used concurrently, thus offering a realistic snapshot of HAP levels and exposures in practice. However, it is essential to interpret these measurements as potentially including other background levels of air pollution from household or ambient sources and thus a possible underestimation of the positive effect of cleaner stoves and clean fuels. The observation that clean fuel interventions consistently achieved the WHO IT-1 level for PM2.5 despite potential stacking of fuels within these households, and potential contributions from PM2.5 in ambient air, is noteworthy and suggests that it may be possible to achieve even lower concentrations and exposure if cleaner technologies and fuels are used exclusively within the community.

The results of these evaluation studies provide a useful guide to the HAP and exposure reductions achieved by different cooking solutions; however, the actual performance in any given setting will vary according to the contextual and operational conditions in which the stove/fuel combinations are used, including the housing type and condition, user-needs, geographical and climatic conditions, and many other factors (Rehfuess et al 2014, Puzzolo et al 2016, Stanistreet et al 2014). Results should therefore be interpreted with caution, with the overall pooled results for each stove and fuel type seen as a general guide to what is being achieved, while individual studies can provide more detailed and context-specific insights.  This concern may be especially important for interpreting the cleanest options, such as LPG and electricity, as the user groups who can afford these fuels for cooking often have substantial socioeconomic and demographic differences from biomass users.  At the same time, there is evidence from before and after studies that biomass-using homes transitioning to LPG can achieve low HAP exposures (Johnson et al 2018).

While a greater number of studies are being reported on LPG interventions, a limitation of the current evidence base is the paucity of studies being published on the intervention impacts of advanced combustion stoves and other clean fuels (e.g. alcohol fuels and electricity). There is also a lack of empirical evidence on the direct effects of interventions on personal exposure to PM2.5. This is likely to partly be the result of an historical lack of suitable technology for measuring personal PM, especially in young children, as well as practical challenges involved including cost, and inconvenience to subjects. Moreover, although there are a few large-scale programmes ongoing that provide evidence on intervention impacts, the majority to date have been studies of relatively small projects and programmes. Given that many of these studies are early efficacy studies typically conducted at the beginning of wider programmes, it is not possible to extrapolate their effectiveness in terms of intervention impacts if implemented at scale over the longer term. The durability of the stove technologies over the longer term needs to be considered in terms of their effectiveness beyond the evaluation periods – there is evidence to suggest, for example, that properly maintained LPG stoves might be more robust in their efficacy to reduce household air pollution over time than more advanced biomass cookstoves (Shen et al 2018, Mortimer et al 2016).  However, given the paucity of evidence further research on long term viability of cleaner cooking options in reducing HAP is needed.

One issue identified through the synthesis of evidence for the review was the presence of potential publication bias identified through statistical funnel plot asymmetry when pooling results.  Such bias could have exaggerated observed intervention impacts for reductions in both PM2.5 and CO due to the pooled evidence being based on studies with larger samples of homes with potentially greater reductions in concentrations. Conversely, estimates derived from the included studies would also include contributions to levels of PM2.5 and CO from other sources (for example other use of solid fuels and kerosene for heating and lighting or from ambient sources that would have diluted the observed intervention impacts from cleaner cooking.

Another consideration is that most studies summarised measures of PM2.5 and CO using arithmetic means leading to the syntheses through meta-analyses adopting this metric. Given that the distributions of these pollutants were generally positively skewed, geometric means or medians could have provided more reliable summary measures with use of means potentially leading to exaggerated intervention effects. An important implication of this limitation is that for interventions that achieved levels of kitchen concentrations or personal exposure at or below WHO Guideline levels, larger fractions of the study populations would have met these targets than is suggested by the skewed mean estimates. This warrants consideration of a universal reporting structure for future HAP measurement studies (e.g. inclusion of geometric and arithmetic means, stratified by primary and secondary cooking fuel types) to minimize measurement bias when pooling PM2.5 exposure data in future meta analyses.

Recommendations for research and policy

There is a need for more studies conducted in the field to strengthen the evidence base for more robust assessment of intervention impacts of clean fuel interventions (more on LPG but in particular ethanol and electricity interventions). Such studies should adopt standardized methods to objectively assess the range of potentially health damaging pollutants, at least assessing concentrations of and exposures to both PM and CO. Monitoring of multiple stoves and other combustion devices in the home and immediate vicinity, and their contributions to levels of HAP and exposure quantified where possible is critical to advancing the evidence-base. Similarly, levels of ambient air pollution resulting from household combustion of neighbouring homes and other sources (which may include garbage burning, burning of crop residue, brick kilns, transport, industry and power generation) should be measured to assess the contribution to household levels of and personal exposure to PM and CO.

At the time of presenting the synthesis for the review, a large observational study was published supporting the main message from the review in relation to the importance of cooking with clean fuels over solid fuels and biomass to achieve reductions in HAP to substantively benefit health when used in everyday settings.  In the PURE-AIR study, data on kitchen and personal PM2.5 were collected from 2541 households and 998 individuals from 120 rural communities in Bangladesh, Chile, China, Columbia, India, Pakistan, Tanzania and Zimbabwe (Shupler et al 2020). The authors found that women from households primarily cooking with electricity (48hr mean = 62 µg/m3 (range=50-76)) or gas (48 µg/m3 (43-54)), although typically with additional ‘stacking’ from secondary fuels had significantly lower exposure levels than those using coal (71 µg/m3 (57-86)), crop waste (97 µg/m3 (73-128)), wood (78 µg/m3 (69-89)), dung (146 µg/m3 (110-194)) and shrubs/ grass (147 µg/m3 (109-197)). Whilst exposure to PM2.5 for women using clean fuels were above the WHO IT-1 Guideline level for PM2.5 of 35 µg/m3, the authors note the often high ambient levels of HAP and suggest that “community-level transition to clean cooking fuels, and potentially emissions reductions from other ambient sources” are required to maximise health gains.

The PURE-AIR study supports the main findings of this analysis, that clean fuel sources are associated with the lowest HAP exposures. In order to bring HAP exposure down to these lower exposure levels across populations, the efforts to characterize effectiveness of interventions and the factors driving contributions to exposure as described above need to be complemented with understanding how those factors can be impacted through policy, pricing, behaviour, and other levers. For example, while venting of stoves is to be encouraged to reduce indoor levels and personal exposures, finding ways to reduce total emissions to the ambient environment (e.g. cleaner combustion technologies or increasing chimney height) is critical to achieving low neighbourhood levels and minimizing exposure of the total population. Similarly, the most effective interventions using clean fuels need to be supported with efforts (e.g. financing mechanisms or comprehensive distribution networks to support continuous household supply of LPG or ethanol) to maximize displacement of traditional technologies. As HAP is influenced by different types of household energy as well as ambient air pollution entering the house, a comprehensive approach towards measuring and addressing HAP is needed. This includes taking heating and lighting into consideration when promoting clean energy use – in addition to cooking.

More high quality and direct feedback of program performance and effectiveness will also help inform decisions and facilitate efforts to maximize exposure reductions. Most critical, the paucity of personal exposure data needs to be addressed. New and more user-friendly instruments for measuring stove usage and exposure are a step in this direction (Burrowes et al 2020, Pillarisetti et al 2019, Wilson et al 2020, Chartier et al 2017, Liao et al 2019, Ventrella et al 2020, Volckens et al 2017), though the management and deployment of these devices still need to be supported by technical know-how. This new technology can enable the design of larger scale studies, such as the PURE-AIR and HAPIN studies (Arku et al 2018, Johnson et al 2020), that include measurements across many countries and/or communities under a harmonized protocol to minimize measurement biases when pooling data across locations in future meta-analyses.

Models to predict HAP exposure have also shown promise to scale-up quantitative exposures to a larger population  (Baumgartner et al 2011, Johnson et al in review (minor revisions), Hill et al 2019, Balakrishnan et al 2013, Sanchez et al 2020, Shupler et al 2018), though additional work is needed to test their performance and reliability across contexts. Biomarkers may also have a role, but need further development to understand how they relate to HAP exposure and health outcomes (Rylance et al 2013, Barr et al 2020).

A public health problem affecting almost half of the world’s population demands the most-effective evidence-based cooking solutions are rolled out on a large-scale. Whilst there is not a single “one size fits all” solution to the HAP problem, a combination of suitable technologies and programmatic approaches can be employed to meet the needs of specific geographical, socio-economic and cultural settings. Innovations in solid fuel stoves to reduce emissions will continue, however, the post-intervention PM levels of advanced combustion stoves presented here suggests that a lot of progress still needs to be made before a ‘healthy’, scalable biomass stove is available. With a growing evidence base demonstrating (i) generally disappointing post-intervention levels of PM achieved from cleaner biomass burning stoves and (ii) the consistent effectiveness in PM reduction for clean fuel interventions such as LPG, ethanol and electricity in achieving emissions consistent with WHO Guideline levels for health, greater emphasis needs to be placed on scaling adoption of cleaner fuels for policy, practice and programmes.

Acknowledgements (TBD)

Funding

This study was commissioned and paid for by the World Health Organization (WHO). The authors have been given permission to publish this article. The authors alone are responsible for the views expressed in this article and they do not necessarily represent the views, decisions or policies of the institutions with which they are affiliated.

References

Alexander D A, Northcross A, Karrison T, Morhasson-Bello O, Wilson N, Atalabi O M, Dutta A, Adu D, Ibigbami T, Olamijulo J, Adepoju D, Ojengbede O and Olopade C O 2018 Pregnancy outcomes and ethanol cook stove intervention: A randomized-controlled trial in Ibadan, Nigeria Environment International 111 152–63

Arku R E, Birch A, Shupler M, Yusuf S, Hystad P and Brauer M 2018 Characterizing exposure to household air pollution within the Prospective Urban Rural Epidemiology (PURE) study Environment International 114 307–17

Balakrishnan K, Ghosh S, Ganguli B, Sambandam S, Bruce N, Barnes D F and Smith K R 2013 Modeling national average household concentrations of PM2.5 from solid cookfuel use for the global burden of disease -2010 assessment: results from cross-sectional assessments in India Environmental Health 12 77

Balakrishnan K, Mehta S, Ghosh S, Johnson M A, Brauer M, Naeher L and Smith K R 2014 WHO Guidelines for Indoor Air Quality: Household Fuel Combustion - Population levels of household air pollution and exposures Online: http://www.who.int/airpollution/guidelines/household-fuel-combustion/Review_5.pdf?ua=1

Barr D B, Puttaswamy N, Jaacks L M, Steenland K, Rajkumar S, Gupton S, Ryan P B, Balakrishnan K, Peel J L, Checkley W, Clasen T, Clark M L and (HAPIN Investigative Team) 2020 Design and Rationale of the Biomarker Center of the Household Air Pollution Intervention Network (HAPIN) Trial Environ Health Perspect 128 047010

Baumgartner J, Schauer J J, Ezzati M, Lu L, Cheng C, Patz J and Bautista L E 2011 Patterns and predictors of personal exposure to indoor air pollution from biomass combustion among women and children in rural China Indoor Air 21 479–88

Berrueta V M, Edwards R D and Masera O R 2008 Energy performance of wood-burning cookstoves in Michoacan, Mexico Renewable Energy 33 859–70

Bonjour S, Adair-Rohani H, Wolf J, Bruce N G, Mehta S, Prüss-Ustün A, Lahiff M, Rehfuess E A, Mishra V and Smith K R 2013 Solid Fuel Use for Household Cooking: Country and Regional Estimates for 1980–2010 Environmental Health Perspectives 121 784–90

Bruce N, de Cuevas R A, Cooper J, Enonchong B, Ronzi S, Puzzolo E, MBatchou B and Pope D 2018 The Government-led initiative for LPG scale-up in Cameroon: Programme development and initial evaluation Energy for Sustainable Development 46 103–10

Bruce N, Dherani M, Liu R, Hosgood H D, Sapkota A, Smith K R, Straif K, Lan Q and Pope D 2015 Does household use of biomass fuel cause lung cancer? A systematic review and evaluation of the evidence for the GBD 2010 study Thorax 70 433–41

Bruce N, Smith K, Balmes J, Pope D, Dherani M, Zhang J, Duan X, Bates M, Lin W, Adair-Rohani H, Mehta S, Cohen A and McCracken J 2014 WHO Indoor Air Quality Guidelines: Household Fuel Combustion Health effects of household air pollution (HAP) exposure Online: https://www.who.int/airpollution/household/guidelines/Review_4.pdf?ua=1

Burnett R, Chen H, Szyszkowicz M, Fann N, Hubbell B, Pope C A, Apte J S, Brauer M, Cohen A, Weichenthal S, Coggins J, Di Q, Brunekreef B, Frostad J, Lim S S, Kan H, Walker K D, Thurston G D, Hayes R B, Lim C C, Turner M C, Jerrett M, Krewski D, Gapstur S M, Diver W R, Ostro B, Goldberg D, Crouse D L, Martin R V, Peters P, Pinault L, Tjepkema M, Donkelaar A van, Villeneuve P J, Miller A B, Yin P, Zhou M, Wang L, Janssen N A H, Marra M, Atkinson R W, Tsang H, Thach T Q, Cannon J B, Allen R T, Hart J E, Laden F, Cesaroni G, Forastiere F, Weinmayr G, Jaensch A, Nagel G, Concin H and Spadaro J V 2018 Global estimates of mortality associated with long-term exposure to outdoor fine particulate matter PNAS 115 9592–7

Burnett R T, Pope C A III, Ezzati M, Olives C, Lim S S, Mehta S, Shin H H, Singh G, Hubbell B, Brauer M, Anderson H R, Smith K R, Balmes J R, Bruce N G, Kan H, Laden F, Prüss-Ustün A, Turner M C, Gapstur S M, Diver W R and Cohen A 2014 An Integrated Risk Function for Estimating the Global Burden of Disease Attributable to Ambient Fine Particulate Matter Exposure Environmental Health Perspectives Online: http://ehp.niehs.nih.gov/1307049/

Burrowes V J, Piedrahita R, Pillarisetti A, Underhill L J, Fandiño‐Del‐Rio M, Johnson M, Kephart J L, Hartinger S M, Steenland K, Naeher L, Kearns K, Peel J L, Clark M L, Checkley W, HAPIN Investigators, Nambajimana A, Verma A, Lovvorn A, Diaz A, Papageorghiou A, Toenjes A, Quinn A, Nizam A, Ryan B, Young B, Barr D, Goodman D, Canuz E, Puzzolo E, McCollum E, Mollinedo E, Majorin F, Ndagijimana F, Rosa G, Thangavel G, Chang H, Fuentes I S P, Miranda J J, Ntivuguruzwa J de D, Uwizeyimana J, Peel J, Sarnat J, Liao J, McCracken J, Rosenthal J, Espinoza J G, Campbell J M, Balakrishnan K, Williams K, Smith K, Mukhopadhyay K, Waller L, Moulton L, Jaacks L, Elon L, Thompson L, Laws M, Chiang M, Howard M, Crocker M, Kirby M, Puttaswamy N, De Leon O, Herrera P, Craik R, Merrick R, Sambandam S, Garg S, Rajkumar S, Gupton S, Hossen S, Sinharoy S, Jabbarzadeh S, Harvey S, Simkovich S, Clasen T, Ramakrishnan U, Davila‐Roman V G, Aravindalochanan V, Chen Y and Sakas Z 2020 Comparison of next‐generation portable pollution monitors to measure exposure to PM2.5 from household air pollution in Puno, Peru Indoor Air ina.12638

Champion W M and Grieshop A P 2019 Pellet-fed gasifier stoves approach gas-stove like performance during in-home use in Rwanda Environ. Sci. Technol. Online: https://doi.org/10.1021/acs.est.9b00009

Chartier R, Phillips M, Mosquin P, Elledge M, Bronstein K, Nandasena S, Thornburg V, Thornburg J and Rodes C 2017 A comparative study of human exposures to household air pollution from commonly used cookstoves in Sri Lanka Indoor Air 27 147–59

Clasen T 2020 Design and Rationale of the HAPIN Study: A Multicountry Randomized Controlled Trial to Assess the Effect of Liquefied Petroleum Gas Stove and Continuous Fuel Distribution Environmental Health Perspectives 11

Dabadge A 2019 Subsidizing connections to the poor Nat Energy 1–2

Delapena S, Piedrahita R, Pillarisetti A, Garland C, Rossanese M E, Johnson M and Pennise D 2018 Using personal exposure measurements of particulate matter to estimate health impacts associated with cooking in peri-urban Accra, Ghana Energy for Sustainable Development 45 190–7

Diekman S, Pope D, Falk H, Ballesteros M, Dherani M, Johnson N, Bryden K, Bruce N and Peck M 2014 WHO Indoor Air Quality Guidelines: Household Fuel Combustion - Burns and Poisoning Online: http://www.who.int/indoorair/guidelines/hhfc/Review_10.pdf?ua=1

Garn J V, Sclar G D, Freeman M C, Penakalapati G, Alexander K T, Brooks P, Rehfuess E A, Boisson S, Medlicott K O and Clasen T F 2017 The impact of sanitation interventions on latrine coverage and latrine use: A systematic review and meta-analysis International Journal of Hygiene and Environmental Health 220 329–40

Granderson J, Sandhu J S, Vasquez D, Ramirez E and Smith K R 2009 Fuel use and design analysis of improved woodburning cookstoves in the Guatemalan Highlands Biomass and Bioenergy 33 306–15

HEI 2020 State of Global Air 2020, Special Report (Boston, MA: Health Effects Institute) Online: https://www.stateofglobalair.org/

Hill L D, Pillarisetti A, Delapena S, Garland C, Pennise D, Pelletreau A, Koetting P, Motmans T, Vongnakhone K, Khammavong C, Boatman M R, Balmes J, Hubbard A and Smith K R 2019 Machine-learned modeling of PM2.5 exposures in rural Lao PDR Science of The Total Environment 676 811–22

Hyman E L 1987 The strategy of production and distribution of improved charcoal stoves in Kenya World Development 15 375–86

IEA, IRENA, UNSD, World Bank and WHO 2020 Tracking SDG 7: The Energy Progress Report (Washington DC: World Bank) Online: https://trackingsdg7.esmap.org/downloads

Johnson M A, Steenland K, Piedrahita R, Clark M L, Pillarisetti A, Balakrishnan K, Peel J L, Naeher L P, Liao J, Wilson D, Sarnat J, Underhill L J, Burrowes V, McCracken J P, Rosa G, Rosenthal J, Sambandam S, de Leon O, Kirby M A, Kearns K, Checkley W, Clasen T and HAPIN Investigators 2020 Air Pollutant Exposure and Stove Use Assessment Methods for the Household Air Pollution Intervention Network (HAPIN) Trial Environ Health Perspect 128 047009

Johnson M and Chiang R A 2015 Quantitative Guidance for Stove Usage and Performance to Achieve Health and Environmental Targets Environmental Health Perspectives Online: http://ehp.niehs.nih.gov/1408681

Johnson M, Edwards R, Ghilardi A, Berrueta V, Gillen D, Frenk C A and Masera O 2009 Quantification of Carbon Savings from Improved Biomass Cookstove Projects Environ. Sci. Technol. 43 2456–62

Johnson M, Piedrahita R, Garland C, Pillarisetti A, Sambandam S, Gurusamy T, Mukhopadhyay K, Balakrishnan K, Leon O F de, Mollinedo E, McCracken J P, Liao J, Burrowes V J, Kephart J, Rio M F D, Majorin F, Rosa G, Kirby M A, Steenland K, Naeher L, Rosenthal J P, Clark M L, Peel J L, Checkley W and Clasen T 2018 Exposures to PM2.5 associated with LPG stove and fuel interventions: Pilot results from the HAPIN Trial

Johnson M, Piedrahita R, Pillariseti A, Shupler M, Menya D, Rossanese M, Delapena S, Penumetcha N, Chartier R, Puzzolo E and Pope D in review (minor revisions) Modeling approaches and performance for estimating personal exposure to household air pollution, a case study in Kenya Indoor Air

Kirby M A, Nagel C L, Rosa G, Zambrano L D, Musafiri S, Ngirabega J de D, Thomas E A and Clasen T 2019 Effects of a large-scale distribution of water filters and natural draft rocket-style cookstoves on diarrhea and acute respiratory infection: A cluster-randomized controlled trial in Western Province, Rwanda PLOS Medicine 16 e1002812

Lacey F G, Henze D K, Lee C J, Donkelaar A van and Martin R V 2017 Transient climate and ambient health impacts due to national solid fuel cookstove emissions PNAS 114 1269–74

Liao J, McCracken J P, Piedrahita R, Thompson L, Mollinedo E, Canuz E, De Léon O, Díaz-Artiga A, Johnson M, Clark M, Pillarisetti A, Kearns K, Naeher L, Steenland K, Checkley W, Peel J and Clasen T F 2019 The use of bluetooth low energy Beacon systems to estimate indirect personal exposure to household air pollution Journal of Exposure Science & Environmental Epidemiology 1–11

Mortimer K, Ndamala C B, Naunje A W, Malava J, Katundu C, Weston W, Havens D, Pope D, Bruce N G, Nyirenda M, Wang D, Crampin A, Grigg J, Balmes J and Gordon S B 2016 A cleaner burning biomass-fuelled cookstove intervention to prevent pneumonia in children under 5 years old in rural Malawi (the Cooking and Pneumonia Study): a cluster randomised controlled trial The Lancet 0 Online: /journals/lancet/article/PIIS0140-6736(16)32507-7/abstract

Nightingale R, Lesosky M, Flitz G, Rylance S J, Meghji J, Burney P, Balmes J and Mortimer K 2018 Noncommunicable Respiratory Disease and Air Pollution Exposure in Malawi (CAPS). A Cross-Sectional Study Am J Respir Crit Care Med 199 613–21

Ochieng C A, Zhang Y, Nyabwa J K, Otieno D I and Spillane C 2020 Household perspectives on cookstove and fuel stacking: A qualitative study in urban and rural Kenya Energy for Sustainable Development 59 151–9

Orr A, Kambombo B, Roth C, Harris D and Doyle V 2015 ADOPTION OF INTEGRATED FOOD-ENERGY SYSTEMS: IMPROVED COOKSTOVES AND PIGEONPEA IN SOUTHERN MALAWI Experimental Agriculture 51 191–209

Pillarisetti A, Carter E, Rajkumar S, Young B N, Benka-Coker M L, Peel J L, Johnson M and Clark M L 2019 Measuring personal exposure to fine particulate matter (PM2.5) among rural Honduran women: A field evaluation of the Ultrasonic Personal Aerosol Sampler (UPAS) Environment International 123 50–3

Pope D, Bruce N, Dherani M, Jagoe K and Rehfuess E 2017 Real-life effectiveness of ‘improved’ stoves and clean fuels in reducing PM2.5 and CO: Systematic review and meta-analysis Environment International 101 7–18

Pope D, Bruce N, Irving G and Rehfuess E 2016 Methodological quality of individual studies was assessed using Liverpool Quality Assessment Tools

Puzzolo E, Pope D, Stanistreet D, Rehfuess E A and Bruce N G 2016 Clean fuels for resource-poor settings: A systematic review of barriers and enablers to adoption and sustained use Environmental Research 146 218–34

Quansah R, Semple S, Ochieng C A, Juvekar S, Armah F A, Luginaah I and Emina J 2017 Effectiveness of interventions to reduce household air pollution and/or improve health in homes using solid fuel in low-and-middle income countries: A systematic review and meta-analysis Environment International 103 73–90

Quinn A K, Bruce N, Puzzolo E, Dickinson K, Sturke R, Jack D W, Mehta S, Shankar A, Sherr K and Rosenthal J P 2018 An analysis of efforts to scale up clean household energy for cooking around the world Energy for Sustainable Development 46 1–10

Rehfuess E A, Puzzolo E, Stanistreet D, Pope D and Bruce N G 2013 Enablers and Barriers to Large-Scale Uptake of Improved Solid Fuel Stoves: A Systematic Review Environmental Health Perspectives Online: http://ehp.niehs.nih.gov/1306639/

Rehfuess E, Pope D, Bruce N, Dherani M, Jagoe K, Naeher L and Noonan C 2014 WHO Indoor Air Quality Guidelines: Household Fuel Combustion - Impacts of interventions on household air pollution concentrations and personal exposure Online: http://www.who.int/indoorair/guidelines/hhfc/Evidence_Review_6.pdf?ua=1

Ronzi S, Orton L, Pope D, Valtorta N K and Bruce N G 2018 What is the impact on health and wellbeing of interventions that foster respect and social inclusion in community-residing older adults? A systematic review of quantitative and qualitative studies Syst Rev 7 Online: https://www.ncbi.nlm.nih.gov/pmc/articles/PMC5789687/

Ruiz-Mercado I and Masera O 2015 Patterns of Stove Use in the Context of Fuel–Device Stacking: Rationale and Implications EcoHealth 12 42–56

Rylance J, Gordon S B, Naeher L P, Patel A, Balmes J R, Adetona O, Rogalsky D K and Martin W J 2013 Household air pollution: a call for studies into biomarkers of exposure and predictors of respiratory disease American Journal of Physiology-Lung Cellular and Molecular Physiology 304 L571–8

Sanchez M, Milà C, Sreekanth V, Balakrishnan K, Sambandam S, Nieuwenhuijsen M, Kinra S, Marshall J D and Tonne C 2020 Personal exposure to particulate matter in peri-urban India: predictors and association with ambient concentration at residence Journal of Exposure Science & Environmental Epidemiology 30 596–605

Shen G, Hays M D, Smith K R, Williams C, Faircloth J W and Jetter J J 2018 Evaluating the Performance of Household Liquefied Petroleum Gas Cookstoves Environ. Sci. Technol. 52 904–15

Shupler M, Godwin W, Frostad J, Gustafson P, Arku R E and Brauer M 2018 Global estimation of exposure to fine particulate matter (PM2.5) from household air pollution Environ Int 120 354–63

Shupler M, Hystad P, Birch A, Miller-Lionberg D, Jeronimo M, Arku R E, Chu Y L, Mushtaha M, Heenan L, Rangarajan S, Seron P, Lanas F, Cazor F, Lopez-Jaramillo P, Camacho P A, Perez M, Yeates K, West N, Ncube T, Ncube B, Chifamba J, Yusuf R, Khan A, Hu B, Liu X, Wei L, Tse L A, Mohan D, Kumar P, Gupta R, Mohan I, Jayachitra K G, Mony P K, Rammohan K, Nair S, Lakshmi P V M, Sagar V, Khawaja R, Iqbal R, Kazmi K, Yusuf S and Brauer M 2020 Household and personal air pollution exposure measurements from 120 communities in eight countries: results from the PURE-AIR study The Lancet Planetary Health 4 e451–62

Stanistreet D, Puzzolo E, Bruce N, Pope D and Rehfuess E 2014 Factors influencing household uptake of improved solid fuel stoves in low- and middle-income countries: a qualitative systematic review International journal of environmental research and public health 11 8228–50

Terrado E N, Eitel B, McCracken J and Charron D 2005 Pilot Commercialization of   Improved Cookstoves in Nicaragua Online: http://www.esmap.org/sites/esmap.org/files/08605NicaraguaCookstovesForWeb.pdf

Tripathi A and Sagar A 2019 Ujjwala 2.0, What should be done next? (New Delhi: Collaborative Clean Air Policy Centre) Online: https://ccapc.org.in/s/Ujjwala-V20-Jun-19b.pdf

Ventrella J, Lefebvre O and MacCarty N 2020 Techno-economic comparison of the FUEL sensor and Kitchen Performance Test to quantify household fuel consumption with multiple cookstoves and fuels Development Engineering 5 100047

WHO 2018a Global Health Observatory data repository: DALYs by country (Geneva: World Health Organization) Online: https://apps.who.int/gho/data/node.main.BODHOUSEHOLDAIRDALYS?lang=en

WHO 2018b Global Health Observatory data repository: Household air pollution burden of disease: deaths (Geneva: World Health Organization) Online: https://www.who.int/data/gho/data/themes/topics/indicator-groups/indicator-group-details/GHO/burden-of-disease---deaths

WHO 2014 WHO Guidelines for Indoor Air Quality: Household Fuel Combustion (Geneva: World Health Organization) Online: http://www.who.int/indoorair/guidelines/hhfc/en/

WHO 2010 WHO guidelines for indoor air quality: selected pollutants (Bonn: World Health Organization Regional Office for Europe)

WHO 2006 WHO guidelines for particulate matter, ozone, nitrogen dioxide and sulfur dioxide (Geneva: World Health Organization Press)

Wilson D L, Williams K N and Pillarisetti A 2020 An Integrated Sensor Data Logging, Survey, and Analytics Platform for Field Research and Its Application in HAPIN, a Multi-Center Household Energy Intervention Trial Sustainability 12 1805

Young B N, Clark M L, Rajkumar S, Benka‐Coker M L, Bachand A, Brook R D, Nelson T L, Volckens J, Reynolds S J, L’Orange C, Good N, Koehler K, Africano S, Osorto Pinel A B and Peel J L 2019 Exposure to household air pollution from biomass cookstoves and blood pressure among women in rural Honduras: A cross‐sectional study Indoor Air 29 130–42

#### References

1.	Hankey S, Sullivan K, Kinnick A, Koskey A, Grande K, Davidson JH, et al. Using objective measures of stove use and indoor air quality to evaluate a cookstove intervention in rural Uganda. Energy for Sustainable Development. 2015;25:67-74.

2.	Ochieng C, Vardoulakis S, Tonne C. Household air pollution following replacement of traditional open fire with an improved rocket type cookstove. The Science of the total environment. 2017;580(uj0, 0330500):440-7.

3.	Rosa G, Majorin F, Boisson S, Barstow C, Johnson M, Kirby M, et al. Assessing the impact of water filters and improved cook stoves on drinking water quality and household air pollution: a randomised controlled trial in Rwanda. PloS one. 2014;9(3):e91011.

4.	Sambandam S, Balakrishnan K, Ghosh S, Sadasivam A, Madhav S, Ramasamy R, et al. Can currently available advanced combustion biomass cook-stoves provide health relevant exposure reductions? Results from initial assessment of select commercial models in India. EcoHealth. 2015;12(1):25-41.

5.	Brant S, Pennise D, Balakrishnan K, Sambandam S, Charron D. Monitoring for Success: Program Evaluation for a New Generation of Biomass Cookstoves. 2012.

6.	Pennise D, Brant S, Agbeve S, Quaye W, Mengesha F, Tadele W. Indoor air quality impacts of an improved wood stove in Ghana and an ethanol stove in Ethiopia. Energy for Sustainable Development. 2009;13(2):71-6.

7.	Dutta K, Naumoff Shields K, Edwards R, Smith K. Impact of improved biomass cookstoves on indoor air quality near Pune, India. . Energy for Sustainable Development 2007;11(2):19-32.

8.	Ochieng C, Vardoulakis S, Tonne C. Are rocket mud stoves associated with lower indoor carbon monoxide and personal exposure in rural Kenya? Indoor Air. 2013;23:14-24.

9.	Pennise D, Haigler E, Ndemere J, Charron D. Indoor Air Pollution Monitoring Summary for Urban Community Development Association’s Wood Stove Project. 2007.

10.	Alexander D, Larson T, Bolton S, Vedal S. Systolic blood pressure changes in indigenous Bolivian women associated with an improved cookstove intervention. Air Quality, Atmosphere and Health. 2015;8(1):47-53.

11.	Binaya KC, Hall IP, Jones B, Shrestha BP. The effect of enhanced stove design on ‘real life’exposure to PM2. 5 and CO in rural dwellings in Salambu, Nepal. 2017(18 April 2019).

12.	Young BN, Clark ML, Rajkumar S, Benka-Coker ML, Bachand A, Brook RD, et al. Exposure to household air pollution from biomass cookstoves and blood pressure among women in rural Honduras: A cross-sectional study. Indoor Air. 2019;29(1):130-42.

13.	Zaman SB, Hossain N, Khandker S. Impact of improved cooking stove on maternal health in rural Bangladesh: A quasi-experimental study. Journal of Medical Research and Innovation. 2017;1(3):1-9.

14.	Albalak R, Bruce N, McCracken J, Smith K, De-Gallardo T. Indoor respirable particulate matter concentrations from an open fire, improved cookstove, and LPG/open fire combination in a rural Guatemalan community. . Environmental Science and Technology. 2001;35(13):2650-5.

15.	Alexander D, Callihan-Linnes J, Bolton S, Larson T. Ventilated cookstoves associated with improvements in respiratory health-related quality of life in rural Bolivia. J Public Health. 2013:1-7.

16.	Bruce N, McCracken J, Albalak R, Schei M, Smith K, Lopez V. The impact of improved stoves, house construction and child location on levels of indoor air pollution and exposure in young Guatemalan children. Journal of Exposure Analysis and Environmental Epidemiology. 2004;14:S110-S7.

17.	Chengappa C, Edwards R, Bagpai R, Naumoff Shields K, Smith K. Impact of improved cookstoves on indoor air quality in the Bundelkhand region in India. . Energy for Sustainable Development. 2007;11:33-44.

18.	Choudhury Z, Le L, Al Masud A, Chang K, Alauddin M, Hossain M. Quantification of indoor air pollution from using cookstoves and estimation of its health effects on adult women in northwest Bangladesh. Aerosol and air quality research. 2012;12:463-75.

19.	Clark M, Bachand A, Heiderscheidt J, Yoder S, Luna B, Volckens J, et al. Impact of a cleaner-burning cookstove intervention on blood pressure in Nicaraguan women. Indoor Air. 2013;23:105-14.

20.	Cowlin S, Kaufmann R, Edwards R, Smith K. Impact of improved stoves on indoor air quality in Ulaanbaatar, Mongolia. Washington, DC: World Bank. 2005.

21.	Cynthia A, Edwards R, Johnson M, Zuk M, Rojas L, Jimenez R. Reduction in personal exposures to particulate matter and carbon monoxide as a result of the installation of a Patsari improved cook stove in Michoacan Mexico. Indoor Air. 2008;18(2):93-105.

22.	Edwards R, Liu Y, He G, Yin Z, Sinton J, Peabody J. Household CO and PM measured as part of a review of China's National Improved Stove Program. Indoor Air. 2007;17:189-203.

23.	ENPHO, Tuladhar B, Gurung I, Shrestha B, Singh A. Final Report on Assessment of Effectiveness of ICS in Reducing IAP and Improving Health. 2008.

24.	Hartinger S, Commodore A, Hattendorf J, Lanata C, Gil A, Verastegui H, et al. Chimney stoves modestly improved Indoor Air Quality measurements compared with traditional open fire stoves: results from a small-scale intervention study in rural Peru. Indoor Air. 2013;23:342-52.

25.	Bates E, Pope D, Bruce N. Smoke, health and household energy Volume 1:Participatory methods for design, installation, monitoring and assessment of smoke alleviation technologies. Rugby UK: ITDG; 2005.

26.	Bates E PD RE, Bruce N. . Smoke, health and household energy Volume 2: Researching pathways to scaling up sustainable and effective kitchen smoke alleviation  Rugby UK: ITDG. 2007.

27.	Lam N, Africano S, Humphrey S, Pennise D. Indoor Air Quality Impact of an Improved Chimney Wood Stove in Peri- Urban Honduras. 2012.

28.	Li Z, Sjödin A, Romanoff L, Horton K, Fitzgerald C, Eppler A. Evaluation of exposure reduction to indoor air pollution in stove intervention projects in Peru by urinary biomonitoring of polycyclic aromatic hydrocarbon metabolites. . Environ International 2011;37:1157-63.

29.	Matsumura K. Improved Stove Evaluation. Boulder, CO.: University of Colorado at Boulder. 2011.

30.	Naeher L, Smith K, Leaderer B, Neufeld L, Mage D. Carbon monoxide as a tracer for assessing exposures to particulate matter in wood and gas cookstove households in highland Guatemala. Environmental Science and Technology

2001;35:575-81.

31.	PAC. Practical Action Consulting: Ethanol as a Household Fuel in Madagascar: Health Benefits, Economic Assessment and Review of African Lessons for Scaling up. Component A - Analysis of Household Air Pollution Interventions in Madagascar. 2011.

32.	Singh A, Tuladhar B, Bajracharya K, Pillarisetti A. Assessment of effectiveness of improved cook stoves in reducing indoor air pollution and improving health in Nepal. . Energy for Sustainable Development. 2012;16:406-14.

33.	Smith K, McCracken J, Thompson L, Edwards R, Shields K, Canuz E, et al. Personal child and mother carbon monoxide exposures and kitchen levels: methods and results from a randomized trial of woodfired chimney cookstoves in Guatemala (RESPIRE). J Expo Sci Environ Epidemiol. 2010;20(5):406-16.

34.	Terrado E, Eitel B. Pilot Commercialization of Improved Cookstoves in Nicaragua. Washington DC: ESMAP. 2005.

35.	Winrock. USAID. Peru Healthy Kitchen/ Healthy Stove Pilot Project. 2008.

36.	Balakrishnan K, Sambandam S, Ghosh S, Mukhopadhyay K, Vaswani M, Arora NK, et al. Household Air Pollution Exposures of Pregnant Women Receiving Advanced Combustion Cookstoves in India: Implications for Intervention. Annals of global health. 2015;81(3):375-85.

37.	Hill LD, Pillarisetti A, Delapena S, Garland C, Jagoe K, Koetting P, et al. Air Pollution and Impact Analysis of a Pilot Stove Intervention Report to the Ministry of Health and Inter--‐Ministerial Clean Stove Initiative of the Lao People’s Democratic Republic Final Report July 20, 2015. Berkeley, California: UC Berkeley Household Energy, Health and Climate Group and the Berkeley Air Monitoring Group; 2015. 2015.

38.	Balakrishnan K. Advanced combustion solid fuel stoves - intervention studies in India 2012.

39.	Bates E, Pope D, Bruce N. Cooking interventions to reduce household air pollution in Sudan. ITDG. 2005.

40.	Delapena S, Piedrahita R, Pillarisetti A, Garland C, Rossanese ME, Johnson M, et al. Using personal exposure measurements of particulate matter to estimate health impacts associated with cooking in peri-urban Accra, Ghana. Energy for Sustainable Development. 2018;45:190-7.

41.	Gautam S, Pillarisetti A, Yadav A, Singh D, Arora N, Smith K. Daily average exposures to carbon monoxide from combustion of biomass fuels in rural households of Haryana, India. Environment, Development and Sustainability. 2018:1-9.

42.	Pokhrel AK, Bates MN, Acharya J, Valentiner-Branth P, Chandyo RK, Shrestha PS, et al. PM<inf>2.5</inf> in household kitchens of Bhaktapur, Nepal, using four different cooking fuels. Atmospheric Environment. 2015;113:159-68.

43.	Pope D, Chartier RT, Higgerson J, Hyseni L, Ronzi S, Mbatchou Ngahane BH, et al. Assessment of Traditional and LPG Stove Use on Household Air Pollution and Personal Exposures in South West Cameroon.2018.

44.	Tagle M, Pillarisetti A, Hernandez MT, Troncoso K, Soares A, Torres R, et al. Monitoring and modeling of household air quality related to use of different Cookfuels in Paraguay. Indoor Air. 2019;29(2):252-62.

45.	Ubuoh EA, Nwajiobi B. Implications of Different Household Cooking Energy on Indoor Air Quality in Urban and Semi-Urban Settlements in Imo, South Eastern Nigeria. Journal of Applied Sciences & Environmental Management. 2018;22(5):725-9.

46.	Rollin H, Mathee A, Bruce NG, Levin J, von Schirnding YER. Comparison of indoor air quality in electrified and un-electrified dwellings in rural South African villages. Indoor Air. 2004;14:208-16.

Figure 1: PRISMA Flow Chart

Note: Original = studies from Pope et al, 2017 (up to 31/12/2015).  Update = additional studies from 1/1/2016..

Figure 2: Forest plot summarizing intervention impacts on levels of kitchen PM2.5 (mg/m3)

New study estimates from updated review indicated with prefix (-)

Figure 3: Forest plot summarizing intervention impacts on levels of kitchen CO (ppm)

New study estimates from updated review indicated with prefix (-)

Figure 4: Forest plot summarizing intervention impacts on levels of personal PM2.5 (mg/m3)

New study estimates from updated review indicated with prefix (-)

Figure 5: Forest plot summarizing intervention impacts on levels of personal CO (ppm)

New study estimates from updated review indicated with prefix (-)

| Figure 6: Pooled weighted mean reductions in household air pollution (PM2.5 and CO) by intervention |  |
| --- | --- |
| n=11 n=6 n=4 n=9 n=2 n=29 n=12 | n=1 n=8 n=2 n=9 |
| Figure 6.1: Kitchen concentrations (PM2.5) | Figure 6.2: Personal exposure (PM2.5) |
| n=2 n=3 n=4 n=7 n=3 n=27 n=13 | n=1 n=2 n=11 n=2 |
| Figure 6.3: Kitchen concentrations (COppm) | Figure 6.4: Personal exposure (COppm) |

Figure 7. Baseline and intervention levels for kitchen PM2.5 (mg/m3) for all improved solid fuel stove and clean fuel interventions in relation to WHO Air Quality Guideline.

Note: Reference line indicates the WHO Air Quality Guideline level (Interim Target 1) for annual PM2.5 of 0.035 mg/m3 (WHO 2006). Interventions 1-4 = cleaner cook stoves and 5-6 = clean fuels.

Figure 8: Baseline and intervention levels for kitchen CO (ppm) for all improved solid fuel stove and cleaner fuel interventions in relation to WHO Air Quality Guideline.

Note: Reference line indicates the WHO Air Quality Guideline (AQG) level for 24-hour CO of 6.11 ppm (WHO 2010). This represents the AQG limit of 7 mg/m3 illustrated, converted to a value of 6.11 ppm using the formula ppm = (mg/m3 value) × (24.45) / (molecular weight = 28.01), and assuming a temperature of 25 °C and a pressure of 1 atm. Interventions 1-4 = cleaner cook stoves and 5-6 = clean fuels.

Figure 9. Baseline and intervention levels for personal exposure to PM2.5 (mg/m3) for all improved solid fuel stove and clean fuel interventions in relation to WHO Air Quality Guideline.

Note: Reference line indicates the WHO Air Quality Guideline level (Interim Target 1) for annual PM2.5 of 0.035 mg/m3 (WHO 2006). Interventions 1-2 = cleaner cook stoves and 3-4 = clean fuels.

Figure 10: Baseline and intervention levels for personal exposure to CO (ppm) for all improved solid fuel stove and cleaner fuel interventions in relation to WHO Air Quality Guideline.

Note: Reference line indicates the WHO Air Quality Guideline (AQG) level for 24-hour CO of 6.11 ppm. Interventions 1-3 = cleaner cook stoves and 4 = clean fuel.

#### Table 1: Summary of included studies: Improved cookstoves without venting (ICS – no vent)

| IMPROVED COOKSTOVES (WITHOUT VENTING) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Author; Year (reference) | Country (season) | Baseline: Intervention | Study design | Duration of follow-up | Kitchen concentration |  | Personal exposure |  | Quality assessment |
|  |  |  |  |  | PM2.5 | CO | PM2.5 | CO |  |
| -Hankey 2015(1) | Uganda (June-August, dry season) | Traditional/ three-stone fire: Rocket wood (Ugastove) | Before and after | 1 month | ✓ 48-hr | ✓ 48-hr | ✗ | ✗ | 5/10 stars (50.0%) |
| -Ochieng 2016(2) | Kenya | Traditional/ wood: Rocket mud stove | Before and after | 10 months | ✗ | ✓ 48-hr | ✗ | ✓ 48-hr | 7/12 stars (58.3%) |
| -Rosa 2014(3) | Rwanda | Traditional/: wood burning rocket stove | RCT | 5 months | ✓ 24-hr | ✗ | ✗ | ✗ | 6/10 stars (60.0%) |
| -Sambandam (Sankar) 2014(4) | India | Traditional/ biomass (mainly wood or dung): Envirofit- B1200 biomass natural-draft rocket stove | Before and after | 1-6 months | ✓ 24-hr | ✓ 24-hr | ✗ | ✗ | 5/10 stars (50.0%) |
|  |  | Traditional/ biomass (mainly wood or dung): Envirofit-G3300 biomass natural-draft rocket stove | Before and after | 1-6 months | ✓ 24-hr | ✓ 24-hr | ✗ | ✗ |  |
|  |  | Traditional/ biomass (mainly wood or dung): Prakti-Leo biomass natural-draft rocket stove | Before and after | 1-6 months | ✓ 24-hr | ✓ 24-hr | ✗ | ✗ |  |
| Brant 2012(5) | India | Traditional/ wood, dung: B1100-rocket | Before and after | 5-6 months | ✓ 24-hr | ✓ 24-hr | ✗ | ✗ | 9/12 stars (75.0%) |
|  |  | Traditional/ wood, dung: Mangala 1-pot |  |  | ✓ 24-hr | ✓ 24-hr | ✗ | ✗ |  |
|  |  | Traditional/ wood, dung: Mangala 2-pot |  |  | ✓ 24-hr | ✓ 24-hr | ✗ | ✗ |  |
|  |  | Traditional/ wood, dung: S2100-rocket |  |  | ✓ 24-hr | ✓ 24-hr | ✗ | ✗ |  |
| CEIHD 2006(6) | Ghana | Traditional/ biomass: Enterprise works/ VITA Gyapa Rocket | Before and after | <6 months | ✓ 24-hr | ✓ 24-hr | ✗ | ✗ | 8 / 12 stars (66.7%) |
| Dutta 2007(7) | India | Traditional/ wood, dung: Bhagyalaxmi | Before and after | 12 months | ✓ 48-hr | ✓ 24-hr | ✗ | ✗ | 8 / 12 stars (66.7%) |
| Ochieng 2013(8) | Kenya | Traditional/ biomass: GIZ mud rocket | Cross-sectional | 10 months | ✓ 48-hr | ✓ 48-hr | ✗ | ✗ | 7/9 stars (77.8%) |
| Pennise 2007(9) | Uganda | Traditional/ wood, dung: UCEODA Rocket | Before and after | 1 month to 1 year | ✓ 24-hr | ✓ 24-hr | ✗ | ✗ | 6 / 12 stars (50.0%) |

#### Table 2: Summary of studies included in the systematic review: Improved cookstoves with venting (ICS – vent)

| IMPROVED COOKSTOVES (WITH VENTING) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Author; Year (reference) | Country (season) | Baseline: Intervention | Study design | Duration of follow-up | Kitchen concentration |  | Personal exposure |  | Quality assessment |
|  |  |  |  |  | PM2.5 | CO | PM2.5 | CO |  |
| -Alexander 2016(10) | Bolivia | Traditional/ biomass: Yanayo stove | Before and after | 12 months | ✓ 24-hr | ✗ | ✗ | ✗ | 8/10 stars (80.0%) |
| -Binaya 2017(11) | Nepal | Traditional/ biomass: 2-pot mud ICS | Cross-sectional | Not described | ✓ 24-hr | ✓ 24-hr | ✓ 24-hr | ✓ 24-hr | 7/11 starts (63.6%) |
| -Young 2018(12) | Honduras | Traditional/ biomass: Rocket with chimney | Cross-sectional | Not described | ✓ 24-hr | ✗ | ✓ 24-hr | ✗ | 8/11 stars (72.7%) |
| -Zaman 2015(13) | Bangladesh | Traditional/ biomass: Earthen stove | Quasi RCT | 6 months | ✗ | ✗ | ✓ 24-hr | ✗ | 4/10 stars (40.0%) |
| Albalak 2001(14) | Guatemala | Traditional/ biomass: Plancha | Cross-sectional | Several years | ✓ PM3.5 24-hr | ✗ | ✗ | ✗ | 5 / 7 stars (71.4%) |
| Alexander et al 2014(15) | Bolivia | Traditional biomass: Yanayo stove | Before and after | 12 months | ✗ | ✓ 48-hr | ✗ | ✗ | 8/12 stars (66.7%) |
| Brant 2012(5) | India | Traditional/ wood, dung: Mangala 2-pot chimney | Before and after | 5-6 months | ✓ 24-hr | ✓ 24-hr) | ✗ | ✗ | 9/12 stars (75.0%) |
| Bruce et al 2004(16) | Guatemala | Traditional/ biomass: Plancha | Cross-sectional | Several years | ✗ | ✓ 24-hr | ✗ | ✓ 24-hr | 8/11 stars (72.7%) |
| Chengappa 2007(17) | India (winter) | Traditional/ wood, dung: DA 2-pot with chimney | Before and after | 12 months | ✗ | ✓ 48-hr | ✗ | ✗ | 7/12 stars (58.3%) |
|  | India (summer) | Traditional/ wood, dung: DA 2-pot with chimney | Before and after | 12 months | ✓ 48-hr | ✓ 48-hr | ✗ | ✗ | 7/12 stars (58.3%) |
| Choudhury 2012(18) | Bangladesh | Traditional/ biomass: BCSIR stove | Cross-sectional | Not described | ✓ 24-hr | ✓ 24-hr | ✗ | ✗ | 7/9 stars (77.8%) |
| Clark et al 2013(19) | Nicaragua | Traditional/ biomass: Prolena eco-stove | Before and after | 9-12 months | ✓ 48-hr | ✓ 48-hr | ✗ | ✓ 48-hr | 9/12 stars (75.0%) |
| Cowlin 2005a(20) | Mongolia | Traditional/ coal: G2-2000 | Cross-sectional | 6+ months | ✓ 24-hr | ✓ 24-hr | ✗ | ✗ | 8 / 9 stars (88.9%) |
|  |  | Traditional/ coal: TT03 |  |  | ✓ 24-hr | ✓ 24-hr | ✗ | ✗ |  |
| Cynthia 2008(21) | Mexico | Traditional/ biomass: Patsari | Before and after | 1 month | ✓ 48-hr | ✓ 24-hr | ✓ 24-hr | ✓ 24-hr | 11/16 stars (68.8%) |
| Dutta 2007(7) | India | Traditional/ wood, dung: ARTI Laxmi | Before and after | 12 months | ✓ 48-hr | ✓ 48-hr | ✗ | ✗ | 8/12 stars (66.7%) |
| Edwards 2007(22) | China | Traditional/ biomass: NISP | Cross-sectional | Several years | ✓ PM4 22 +/- 1 hr (>20) | ✓ 24-hr | ✗ | ✗ | 6/9 stars (66.7%) |
| ENPHO 2008a(23) | Dolakha Nepal | Traditional/ biomass: Mudbrick ICS | Before and after | < 3 months | ✓ 24-hr | ✓ 24-hr | ✗ | ✗ | 7/12 stars (58.3%) |
| ENPHO 2008b(23) | Dang Nepal | Traditional/ biomass: Mudbrick ICS | Before and after | < 3 months | ✓ 24-hr | ✓ 24-hr | ✗ | ✗ | 7/12 stars (58.3%) |
| ENPHO 2008c(23) | Ilan Nepal | Traditional/ biomass: Mudbrick ICS | Before and after | < 3 months | ✓ 24-hr | ✓ 24-hr | ✗ | ✗ | 7/12 stars (58.3%) |
| Hartinger et al 2013(24) | Peru | Traditional/ biomass: Optima | Cross-sectional | 6-8 months | ✓ 48-hr | ✓ 48-hr | ✓ 48-hr | ✓ 48-hr | 8/11 stars (72.7%) |
| ITDG 2005(25) | Nepal (winter) | Traditional/ biomass: Smoke hood | Before and after | 12 months | ✓ 24-hr | ✓ 24-hr | ✗ | ✗ | 10/14 stars (71.4%) |
|  | Nepal (summer) | Traditional/ biomass: Smoke hood | Before and after | 12 months | ✓ 24-hr | ✓ 24-hr | ✗ | ✗ | 10/14 stars (71.4%) |
| ITDG 2007(26) | Nepal | Traditional/ biomass: Smoke hood | Before and after | 6-9 months | ✗ | ✓ 24-hr | ✗ | ✗ | 10/14 stars (71.4%) |
| Lam 2012(27) | Honduras | Traditional/ biomass: ECO biomass stove | Before and after | 3 weeks | ✓ 24-hr | ✗ | ✗ | ✗ | 8/12 stars (66.7%) |
| Li 2011(28) | Peru (winter) | Traditional/ biomass: HNP 3-pot metal stove | Before and after | 3 weeks | ✓ 48-hr | ✓ 24-hr | ✓ 24-hr | ✓ 24-hr | 8/12 stars (66.7%) |
|  |  | Traditional/ biomass: BGC 3-pot metal stove |  |  | ✓ 48-hr | ✓ 24-hr | ✓ 24-hr | ✓ 24-hr |  |
| Matsumura 2011(29) | Peru | Traditional/ biomass: GIZ 2-pot design | Cross-sectional | 1-2 months | ✓ 24-hr | ✓ 24-hr | ✗ | ✗ | 3/9 stars (33.3%) |
| Naeher 2001(30) | Guatemala | Traditional/ biomass: Plancha | Cross-sectional | Several years | ✓ 24-hr | ✓ 24-hr | ✗ | ✓ 24-hr (mothers and children) | 5 / 9 stars (55.6%) |
| PAC 2011(31) | Madagascar | Traditional/ biomass: Fatana pipa 1-pot | Before and after | 5 months | ✓ 24-hr | ✓ 24-hr | ✗ | ✗ | 13/14 stars (92.9%) |
| Singh 2012(32) | Nepal | Traditional/ biomass: 2-pot mud ICS | Before and after | 12 months | ✓ 24-hr | ✓ 24-hr | ✗ | ✗ | 7/12 stars (58.3%) |
| Smith et al 2011(33) | Guatemala | Traditional/ biomass: Plancha | RCT | 18 months | ✗ | ✓ 24-hr | ✗ | ✓ 48-hr (mothers and childen) | 12/12 stars (100%) |
| Terrado 2005a(34) | Nicaragua | Traditional/ biomass: Ecostove (closed) | Before and after | 1-2 months | ✓ 24-hr | ✗ | ✓ 24-hr | ✗ | 9/12 stars (75.0%) |
|  |  | Traditional/ biomass: Ecostove (semi-closed) |  |  | ✓ 24-hr | ✗ | ✓ 24-hr | ✗ |  |
| Winrock 2008(35) | Peru | Traditional/ biomass: Rocket with chimney | Before and after | 12 months | ✓ 24-hr | ✗ | ✗ | ✗ | 6/12 stars (50.0%) |

#### Table 3: Summary of studies included in the systematic review: Complex interventions

| COMPLEX INTERVENTIONS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Author; Year (reference) | Country (season) | Baseline: Intervention | Study design | Duration of follow-up | Kitchen concentration |  | Personal exposure |  | Quality assessment |
|  |  |  |  |  | PM2.5 | CO | PM2.5 | CO |  |
| ITDG 2005(39) | Kenya (wet season) | Traditional/ biomass: Hood, LPG, improved, vent, fireless cookstove | Before and after | 12 months | ✓ PM4 24-hr | ✓ 24-hr | ✗ | ✗ | 10 /14 stars (71.4%) |
|  | Kenya (dry season) | Traditional/ biomass: Hood, LPG, improved, vent, fireless cookstove | Before and after | 12 months | ✓ PM4 24-hr | ✓ 24-hr | ✗ | ✗ | 10 /14 stars (71.4%) |

#### Table 4: Summary of studies included in the systematic review: Advanced combustion stoves

| ADVANCED COMBUSTION SOLID FUEL STOVES |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Author; Year (reference) | Country (season) | Baseline: Intervention | Study design | Duration of follow-up | Kitchen concentration |  | Personal exposure |  | Quality assessment |
|  |  |  |  |  | PM2.5 | CO | PM2.5 | CO |  |
| - Balakrishnan 2015(36) | India (Summer) | Traditional/ wood, dung: Phillips-HD4012-Forced Draft biomass micro gasifier stove | Before and after | 9-12 months | ✓ 24-hr | ✓ 24-hr | ✓ 24-hr | ✓ 24-hr | 5/12 stars (41.7%) |
|  | India (Winter) | Traditional/ wood, dung: Phillips-HD4012-Forced Draft biomass micro gasifier stove |  | 1-3 months | ✓ 24-hr | ✓ 24-hr | ✓ 24-hr | ✓ 24-hr |  |
| -Hill 2015(37) | Lao People’s Democratic Republic | Traditional/ biomass: ACE 1 Ultra-Clean Biomass Cookstove | Before and after | 2 weeks | ✓ 48-hr | ✗ | ✓ 48-hr | ✗ | 7/12 stars (58.3%) |
| -Sambandam (Sankar) 2014(4) | India | Traditional/ biomass (mainly wood or dung): Phillips-Natural Draft biomass micro gasifier stove | Before and after | 1-6 months | ✓ 24-hr | ✓ 24-hr | ✗ | ✗ | 5/10 stars (50.0%) |
|  |  | Traditional/ biomass (mainly wood or dung): Phillips-HD4012-Forced Draft biomass micro gasifier stove | Before and after | 1-6 months | ✓ 24-hr | ✓ 24-hr | ✗ | ✗ |  |
|  |  | Traditional/ biomass (mainly wood or dung): Oorja biomass forced-draft micro gasifier stove | Before and after | 1-6 months | ✓ 24-hr | ✓ 24-hr | ✗ | ✗ |  |
| Balakrishnan 2012(38) | India | Traditional/ biomass: Free convection | Before and After | Not available | ✓ ns | ✓ ns | ✗ | ✗ | 8 / 12 stars (66.7%) |
|  |  | Traditional/ biomass: Forced convection/ optimised fuel |  |  | ✓ ns | ✓ ns | ✗ | ✗ |  |
|  |  | Traditional/ biomass: Forced convection |  |  | ✓ ns | ✓ ns | ✗ | ✗ |  |

#### Table 5: Summary of studies included in the systematic review: Ethanol stoves

| ETHANOL STOVES |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Author; Year (reference) | Country (season) | Baseline: Intervention | Study design | Duration of follow-up | Kitchen concentration |  | Personal exposure |  | Quality assessment |
|  |  |  |  |  | PM2.5 | CO | PM2.5 | CO |  |
| CEIHD 2007a(9) | Ethiopia, Kebribeyah | Traditional/ biomass: Ethanol Stove | Before and after | Not specified | ✓ 24-hr | ✓ 24-hr | ✗ | ✗ | 7/12 stars (58.3%) |
| CEIHD 2007b(9) | Ethiopia, Bonga | Traditional/ biomass: Ethanol Stove | Before and after | Not specified | ✓ 24-hr | ✓ 24-hr | ✗ | ✗ | 7/12 stars (58.3%) |
| PAC 2011a(31) | Madagascar, Vatomandry | Traditional/ biomass: Cleancook ethanol Stove | Before and after | 5 months | ✓ 24-hr | ✓ 24-hr | ✗ | ✗ | 13/14 stars (92.9%) |
| PAC 2011b(31) | Madagascar, Ambositra | Traditional/ biomass: Cleancook Ethanol Stove | Before and after | 7 months | ✓ 24-hr | ✓ 24-hr | ✗ | ✗ | 13/14 stars (92.9%) |

#### Table 6: Summary of studies included in the systematic review: LPG stoves

| LIQUIFIED PETROLEUM GAS (LPG) INTERVENTIONS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Author; Year (reference) | Country (season) | Baseline: Intervention | Study design | Duration of follow-up | Kitchen concentration |  | Personal exposure |  | Quality assessment |
|  |  |  |  |  | PM2.5 | CO | PM2.5 | CO |  |
| -Delapana 2017(40) | Ghana | Traditional/ wood: LPG | Cross-sectional | Not specified | ✓ 48-hr | ✗ | ✗ | ✗ | 4/9 stars (44.4%) |
| -Gautam 2008(41) | India | Traditional/ biomass: LPG | Cross-sectional | Not specified | ✗ | ✗ | ✗ | ✓ 24-hr | 6/9 stars (66.7%) |
| -HAPIN 2020a | Guatemala | Traditional/ biomass: LPG | RCT | 2 months | ✓ 24-hr | ✗ | ✓ 24-hr | ✗ | 8/12 stars (66.7%) |
| -HAPIN 2020b | India | Traditional/ biomass: LPG | RCT | 2 months | ✓ 24-hr | ✗ | ✓ 24-hr | ✗ | 8/12 stars (66.7%) |
| -HAPIN 2020c | Rwanda | Traditional/ biomass: LPG | RCT | 2 months | ✗ | ✗ | ✓ 24-hr | ✗ | 8/12 stars (66.7%) |
| -Lewis 2017 | India | Traditional/ biomass: LPG | Cross-sectional | Not-specified | ✓ 24-hr | ✗ | ✗ | ✗ | 6/9 stars (66.7%) |
| -Lewis 2019 | India | Traditional/ biomass: LPG | Cross-sectional | Not specified | ✓ 24-hr | ✗ | ✓ 24-hr | ✗ | 9/11 stars (81.8%) |
| -Pokhrel 2015(42) | Nepal | Traditional/ biomass: LPG | Cross-sectional | Not specified | ✓ 24-hr | ✗ | ✗ | ✗ | 5/9 stars (55.6%) |
| -Pope 2018(43) | Cameroon | Traditional/ wood: LPG | Cross-sectional | Not specified | ✓ 48-hr | ✗ | ✓ 48-hr | ✗ | 9/11 stars (81.8%) |
| -Tagle 2019(44) | Paraguay | Traditional/ wood: LPG | Cross-sectional | Not specified | ✓ 24-hr | ✓ 24-hr | ✗ | ✗ | 7/9 stars (77.8%) |
| -Ubuoh 2018(45) | Nigeria | Traditional/ wood: LPG | Cross-sectional | Not specified | ✓ 24-hr | ✗* | ✗ | ✗ | 5/11 stars (45.5%) |
| ITDG 2005(39) | Sudan (wet) | Traditional/ biomass: LPG | Before and after | 12 months | ✓ 24-hr | ✓ 24-hr | ✗ | ✗ | 10/14 stars (71.4%) |
|  | Sudan (dry) | Traditional/ biomass: LPG | Before and after | 12 months | ✓ 24-hr | ✓ 24-hr | ✗ | ✗ | 10/14 stars (71.4%) |

* Estimates for CO could not be used as expressed as mg/m3 in the paper.

#### Table 7: Summary of studies included in the systematic review: Electric stoves

| ELECTRIC STOVES |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Author, Year (reference) | Country (season) | Baseline: Intervention | Study design | Duration of follow-up | Kitchen concentration |  | Personal exposure |  | Quality assessment |
|  |  |  |  |  | PM2.5 | CO | PM2.5 | CO |  |
| -Lewis 2019 | India | Traditional/ biomass: electric stove | Cross-sectional | Not specified | ✓ 24-hr | ✗ | ✓ 24-hr | ✗ | 9/11 stars (81.8%) |
| -Pokhrel 2015(42) | Nepal | Traditional/ wood: electric stove | Cross-sectional | Not specified | ✓ 24-hr | ✗ | ✗ | ✗ | 5/9 stars (55.6%) |
| -Tagle 2019(44) | Paraguay | Traditional/ wood: electric stove | Cross-sectional | Not specified | ✓ 24-hr | ✓ 24-hr | ✗ | ✗ | 7/9 stars (77.8%) |
| -Ubuoh 2018(45) | Nigeria | Traditional/ wood: electric stove | Cross-sectional | Not specified | ✓ 24-hr | ✓ 24-hr | ✗ | ✗ | 6/11 stars (54.5%) |
| Rollin 2004(46) | South Africa Summer (dry) | Traditional/ kerosene/solid fuel: electrification | Cross-sectional | n/a | ✓ 24-hr | ✓ 24-hr | ✗ | ✗ | 6/9 stars (66.7%) |